Page 1.1

Contents

| Description | Page |
| --- | --- |
| Content page | 1.1 |
| Review of BMR | 1.2a |
| Product Approval for Release (QA Checklist) | 1.2b |
| Product Information Page | 1.3 |
| Product Formulation | 1.4 |
| Special Precautions & Remarks | 1.5 |
| Weighing Area Checklist | 2.1 |
| Batch Weighing Record | 2.2 |
| Granulation Area Checklist | 3.1 |
| Blending/Granulation Record | 3.2 |
| Binding Agent Record | 3.3 |
| Blending/Granulation Procedure | 3.4a |
| Dry Mixing Report | 3.4b |
| Wet Mixing Report | 3.4c |
| Drying Report | 3.4d |
| Milling Report | 3.4e |
| Dried Granules LOD Results | 3.4f |
| Dried Granules Details | 3.4g |
| In-Process QC Report | 3.4h |
| Blending Area Checklist | 3.4i |
| Blending Record | 3.4j |
| Lubricated Granule Details | 3.4k |
| In-Process QC Report | 3.4l |
| Capsulation/Tabletting Area Checklist | 4.1 |
| Capsulation/Tabletting Report | 4.2 |
| In-Process Specification | 4.3a |
| In-Process Check | 4.3b |
| Capsulation/Tabletting Record | 4.4 |
| Weighing Area Checklist – Film Coating | 5.1 |
| Batch Weighing Record | 5.2 |
| Film Coating Area Checklist | 5.3 |
| Preparation Record | 5.4a |
| Film Coating Liquid Preparation Procedures | 5.4b |
| Film Coating Specification | 5.5 |
| Film Coating In-Process Checks | 5.5a |
| Film Coating Record | 5.6a |
| Film Coating Record | 5.6b |
| Finished Product Specification | 5.7 |
| LL Store Area Checklist | 6.0 |
| Unit Box Requisition Form | 6.1a |
| Reference Unit Box | 6.1b |
| Approved Coded Unit Box | 6.1c |
| Packing Area Checklist | 7.1 |
| Blister Packing Procedure | 7.2 |
| Packing Procedure (Visual Inspection) | 7.2a |
| Packing Record | 7.3 |
| Reference Blister | 7.4a |
| Approved Blister Artwork | 7.4b |
| Approved Blister Strip | 7.4c |
| Packing Report | 7.5 |
| Finished Product Records | 7.6 |
| Physical Checked on Packed Products (Production) | 7.7a |
| Physical Checked on Packed Products (QC) | 7.7b |
| Physical Checked on Packed Products (QC) | 7.7c |
| Physical Checked on Packed Products (QC) | 7.7d |
| Reconciliation (Packaging Materials) | 7.8 |
| Yield Report | 7.9 |
| Signature Page | 8.0 |
| Revision History | 9.0 |
| Sticker Page | Appendix 1 |
| Output Labels | Appendix 2 |
| Certificate Of Analysis (Finished Products) | Appendix 3 |
| Microbial Contamination Test Result | Appendix 4 |
| Raw Data for Physical Test | Appendix 5 |
| QC Analytical Reports | Appendix 6 |
| Production Line Clearance Forms | Appendix 7 |
| Finished Product Quarantine Labels | Appendix 8 |
| Ticket Consumptions | Appendix 9 |
| Transfer Shipments | Appendix 10 |
| Transfer Receipt | Appendix 11 |

Page 1.1

Page 1.2a

Review of BMR             QC Ref No.: _____________

| Checklist |  |  | Production |  | Quality Control |  | Quality Assurance |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Yes | No | Yes | No | Yes | No |
| General Information | Cover Page & Content Page |  |  |  |  |  |  |  |
|  | BMR Checklist |  |  |  |  |  |  |  |
|  | Production Information Page & Formulation |  |  |  |  |  |  |  |
|  | Weighing Area Checklist & Batch Weighing Record |  |  |  |  |  |  |  |
|  | Special Precautions and Remarks |  |  |  |  |  |  |  |
| Granulation | Granulation Area Checklist |  |  |  |  |  |  |  |
|  | Granulation Procedures & Record & Dry Mixing Report |  |  |  |  |  |  |  |
|  | Binding Agent Record &Wet Mixing Report |  |  |  |  |  |  |  |
|  | Drying Report & Granules Details |  |  |  |  |  |  |  |
|  | QC Report (Granules) |  |  |  |  |  |  |  |
| Lubrication | Blending Area Checklist & Blending Record |  |  |  |  |  |  |  |
|  | QC Report (Lubrication) |  |  |  |  |  |  |  |
| Tabletting/ Capsulation | Capsule Requisition |  |  |  |  |  |  |  |
|  | Tabletting/capsule Area Checklist |  |  |  |  |  |  |  |
|  | Tabletting/capsule Report and Records |  |  |  |  |  |  |  |
|  | Tabletting/capsule Specifications and in Process Checks |  |  |  |  |  |  |  |
| Film Coating | Film Coating Area Checklist |  |  |  |  |  |  |  |
|  | Film Coating Preparation Procedure and Preparation Record |  |  |  |  |  |  |  |
|  | Film Coating Specification and in Process Check |  |  |  |  |  |  |  |
|  | Film Coating Record |  |  |  |  |  |  |  |
| Sugar Coating | Sugar Coating Area Checklist |  |  |  |  |  |  |  |
|  | Sugar Coating Preparation Procedure and Preparation Record |  |  |  |  |  |  |  |
|  | Sugar Coating Specification and in Process Checks |  |  |  |  |  |  |  |
|  | Sugar Coating Record |  |  |  |  |  |  |  |
| FC/SC | Finished Product Specification (FC/SC) |  |  |  |  |  |  |  |
| Label/ Leaflet/ Unit Box | LL Store Area Checklist |  |  |  |  |  |  |  |
|  | Label, Leaflet, Unit Box Requisition Form |  |  |  |  |  |  |  |
|  | Reference/Approved Label, Leaflet, Unit Box |  |  |  |  |  |  |  |
| Checklist |  |  | Production |  | Quality Control |  | Quality Assurance |  |
|  |  |  | Yes | No | Yes | No | Yes | No |
| Packing |  | Packing Area Checklist |  |  |  |  |  |  |
|  |  | Packing Procedure & Record & Report |  |  |  |  |  |  |
|  |  | Finished Product Record |  |  |  |  |  |  |
|  |  | Physical Check on Packed Products |  |  |  |  |  |  |
|  |  | Reconciliation of Materials |  |  |  |  |  |  |
| Documentation |  | Yield Report, Signature Report, Revision History |  |  |  |  |  |  |
| Appendix |  | Appendix provided by Production |  |  |  |  |  |  |
|  |  | Consumption Labels |  |  | N.A |  |  |  |
|  |  | Output Labels |  |  |  |  |  |  |
|  |  | In-House Labels |  |  |  |  |  |  |
|  |  | Ticket Consumption |  |  |  |  |  |  |
|  |  | Transfer Shipment |  |  |  |  |  |  |
|  |  | Transfer Receipt |  |  |  |  |  |  |
|  |  | Line Clearance Form |  |  |  |  |  |  |
|  |  | Reprocess or Rework form (if any) |  |  |  |  |  |  |
|  |  | Appendix provided by QC |  |  |  |  |  |  |
|  |  | Certificate of Analysis | N.A. |  |  |  |  |  |
|  |  | Microbial Contamination Test Result |  |  |  |  |  |  |
|  |  | Raw Data for Physical Test |  |  |  |  |  |  |
|  |  | Analytical Reports |  |  |  |  |  |  |
|  |  | Quarantine Labels |  |  |  |  |  |  |
|  |  | OOS if any |  |  |  |  |  |  |
| Reviewed By |  |  |  |  |  |  |  |  |
| Date |  |  |  |  |  |  |  |  |

Page 1.2a

Note: Tick (√) Yes/No where applicable, N/A is not applicable.

Page 1.2b

PRODUCT APPROVAL FOR RELEASE (QUALITY ASSURANCE CHECKLIST)

| √ Correct/ complete and available, N/A if not applicable |  |  |  |
| --- | --- | --- | --- |
| No. | Details |  | Status |
| 1 | Batch record and all attached forms are current and approved |  |  |
| 2 | All batch records are signed properly and complying all parameters as per manufacturing instructions |  |  |
| 3 | Specified and released components are used in manufacturing |  |  |
| 4 | Specified qualities of components are used in manufacturing |  |  |
| 5 | Specified product is packed |  |  |
| 6 | Specified packaging components are used |  |  |
| 7 | Labelling bears the specified control number and product information |  |  |
| 8 | Yields are within the limits |  |  |
| 9 | In-process results and finished product results are within the specifications |  |  |
| 10 | Verify final process testing results |  |  |
| 11 | Retention samples and Stability samples as appropriate are taken |  |  |
| 12 | All deviations including OOS investigations associated with the batch are closed and listed below. All remedial actions have been performed. |  |  |
| 13 | Changes related to the batch are approved and the recommendations in change control request are compiled. |  |  |
| 14 | All utilities are within specifications. |  |  |
| 15 | Process parameters end point (e.g. blending time, compression speed etc.) are within the limits specified in manufacturing instructions. |  |  |
| 16 | Batch production record is complete and signed by appropriate Production and Quality Control personnel. |  |  |
| List of all Deviations, OOS investigations and Change Controls: |  |  |  |
| This batch has been manufactured and tested in compliance with the current good manufacturing practices and conforms to specifications. This batch processing, packaging and analysis records were reviewed and found to be in compliance with current good manufacturing practices. |  |  |  |
| Approved |  |  |  |
| Rejected |  |  |  |
| Authorized By: |  |  |  |
| Signature: |  |  |  |
| Date: |  |  |  |

Page 1.3

PRODUCT INFORMATION PAGE

ACTIVE INGREDIENTS 	Metformin HCI 250mg/ tab

--

--

REGISTRATION STATUS

COUNTRY	REG. NO.	PACK SIZE/ SHELF-LIFE 	REMARKS

SINGAPORE	SIN7751P	1,000’s/ 10x10x10’s (5yrs)	--

MALAYSIA	--	--		--

VIETNAM	--	--		--

HONG KONG	HK-61469	100x10’s (5yrs)		--

MYANMAR	--	--		--

PHILLIPPINES	--	--		--

DESCRIPTION

SIZE	10mm			COATING	Film-Coating

SHAPE	Round, convex			THICKENSS	4.5 – 5.2mm

MARKINGS	score			UNIT WEIGHT	359.79 mg ± 5% (core)

COLOUR	White			UNIT WEIGHT	368.42 mg ± 5% (coated)

PACKING

TYPE	SHELF LIFE	PACK SIZE	PACKING MATERIAL

Bulk 	5 years		1,000’s	1 Litre White HDPE rect. bottle	1 bag of silica gel

Blister	5 years		10x10x10’s 	Printed Alum. Foil			Clear PVC Foil

Blister       5 years	        100x10’s 	      Printed Alum. Foil			Clear PVC Foil

STORAGE	Store in cool dry place.

Page 1.4

Product Formulation

| Raw Materials | Unit Weight Per tablet | Excess (%) | Weight for 1,050,000 ’s |
| --- | --- | --- | --- |
| Metformin HCI | 250 mg | -- | 262,500 g |
| Lactose USP 200 mesh | 75 mg | -- | 78,750 g |
| Corn Starch | 25 mg | -- | 26,250 g |
| Plasdone K90 USP (PVP K90) | 3 mg | -- | 3,150 g |
| Kollidon 90F | 5 mg | -- | 5,250 g |
| Magnesium Stearate | 1.79 mg | -- | 1,880 g |
| Isopropyl Alcohol | 0.023 ml | -- | 24,000 ml |
| Purified Water | 0.017 ml | -- | 18,000 ml |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | 359.79 mg | -- | 377,780.00 g |

Note: The granulation stage is divided into three (3) equal sub-lots.

Page 1.4

Product Formulation

| Raw Materials | Unit Weight Per tablet | Excess (%) | Weight for 350,000 ’s |
| --- | --- | --- | --- |
| Sub-Lot 1 | -- | -- | -- |
| Metformin HCI | 250 mg | -- | 87,500 g |
| Lactose USP 200 mesh | 75 mg | -- | 26,250 g |
| Corn Starch | 25 mg | -- | 8,750 g |
| Plasdone K90 USP (PVP K90) | 3 mg | -- | 1,050 g |
| Kollidon 90F | 5 mg | -- | 1,750 g |
| Magnesium Stearate | 1.79 mg | -- | 626.67 g |
| Isopropyl Alcohol | 0.023 ml | -- | 8,000 ml |
| Purified Water | 0.017 ml | -- | 6,000 ml |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | 359.79 mg | -- | 125,926.67 g |

Page 1.4

Product Formulation

| Raw Materials | Unit Weight Per tablet | Excess (%) | Weight for 350,000 ’s |
| --- | --- | --- | --- |
| Sub-Lot 2 | -- | -- | -- |
| Metformin HCI | 250 mg | -- | 87,500 g |
| Lactose USP 200 mesh | 75 mg | -- | 26,250 g |
| Corn Starch | 25 mg | -- | 8,750 g |
| Plasdone K90 USP (PVP K90) | 3 mg | -- | 1,050 g |
| Kollidon 90F | 5 mg | -- | 1,750 g |
| Magnesium Stearate | 1.79 mg | -- | 626.67 g |
| Isopropyl Alcohol | 0.023 ml | -- | 8,000 ml |
| Purified Water | 0.017 ml | -- | 6,000 ml |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | 359.79 mg | -- | 125,926.67 g |

Page 1.4

Product Formulation

| Raw Materials | Unit Weight Per tablet | Excess (%) | Weight for 350,000 ’s |
| --- | --- | --- | --- |
| Sub-Lot 3 | -- | -- | -- |
| Metformin HCI | 250 mg | -- | 87,500 g |
| Lactose USP 200 mesh | 75 mg | -- | 26,250 g |
| Corn Starch | 25 mg | -- | 8,750 g |
| Plasdone K90 USP (PVP K90) | 3 mg | -- | 1,050 g |
| Kollidon 90F | 5 mg | -- | 1,750 g |
| Magnesium Stearate | 1.79 mg | -- | 626.67 g |
| Isopropyl Alcohol | 0.023 ml | -- | 8,000 ml |
| Purified Water | 0.017 ml | -- | 6,000 ml |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | -- -- | -- | -- -- |
| -- | 359.79 mg | -- | 125,926.67 g |

Page 1.5

Special Precautions & Remarks

N/A

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

Page 3.1

Granulation Area Checklist

| GMP Checklist: |  |  |  |  | Date: | Time: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GRANULATION DEPARTMENT |  |  |  | Tick() where appropriate |  | Sign & Date By |  |  |
|  |  |  |  |  |  | Granulation Personnel |  | Production Controller |
| Work Area Check | Materials from previous batch are removed |  |  | □ No | □ Yes |  |  |  |
|  | Room name: |  |  | □ Unclean | □ Clean |  |  |  |
|  | Room status labelled |  |  | □ No | □ Yes |  |  |  |
|  | Status of Utensils |  |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Vacuum cleaner |  |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Mechanical Sifter |  |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Super Mixer |  |  | □ Unclean | □ Clean |  |  |  |
|  | Status of FBD |  |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Power mil |  |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Exhaust |  |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Air-con |  |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Weighing balance |  |  | □ Unclean | □ Clean |  |  |  |
|  | All equipment are labelled |  |  | □ No | □ Yes |  |  |  |
|  | Gloves and PPE worn by operators |  |  | □ No | □ Yes |  |  |  |
|  | Previous Item granulated & Batch No : |  |  |  |  |  |  |  |
| Room Condition | Room Temperature (not more than 25degC) (ID.No.:________) |  |  | Relative Humidity (not more than 75%) |  |  |  |  |
| Equipment Check | Sieving: | Sieve Size |  | □ #40_____ □ #100 ____ |  |  |  |  |
|  |  | Mechanical Sifter |  | □ EQP ID : SF-01 |  |  |  |  |
|  | Granulation: | Super Mixer |  | □ EQP ID: BL-07 |  |  |  |  |
|  |  | Power Mixer |  | □ EQP ID: _________ |  |  |  |  |
|  | Drying & Milling: | Fluid Bed Dryer |  | □ EQP ID: DR-03 (Finger Bag ID No.:____) |  |  |  |  |
|  |  | Power Mill Screen Size (ID.No.) |  | □ EQP ID: PML-03 |  |  |  |  |
|  |  |  |  | □ 2.0 mm □ Other: ____________ |  |  |  |  |
|  |  | Milling Speed |  | □ High □ Medium □ Low |  |  |  |  |
|  | Electronic Balance (60 Kgs) |  |  | □ EQ No:________ |  |  |  |  |

Note: Production should not proceed unless all checks comply.

* To Production Personnel: Please be reminded to report immediately to the production management team whenever there is any manufacturing deviation.

Line Clearance given by (Production Supervisor):_______________ Date: __________________

Page 3.2

Blending/Granulation Record

Batch Size: 1,050,000’S

| Raw Materials | Theoretical Wt 350,000’s | Actual Weight 350,000’s | Signed & Date |  |
| --- | --- | --- | --- | --- |
|  |  |  | Received By | Checked By |
| Sub Lot 1 | -- -- | -- | -- | -- |
| Metformin HCI | 87.5 kg | kg |  |  |
| Lactose USP 200 mesh | 26.25 kg | kg |  |  |
| Corn Starch | (8,750g) 8.75kg | kg |  |  |
| Plasdone K90 USP (PVP K90) | (1,050g) 1.05 kg | kg |  |  |
| Kollidon 90F | (1,750g) 1.75 kg | kg |  |  |
| Isopropyl Alcohol | (8L) 6.30 kg | kg |  |  |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |

________________	________________	Production Controller	Date of Issue of raw materials

Page 3.2

Blending/Granulation Record

Batch Size: 1,050,000’S

| Raw Materials | Theoretical Wt 350,000’s | Actual Weight 350,000’s | Signed & Date |  |
| --- | --- | --- | --- | --- |
|  |  |  | Received By | Checked By |
| Sub Lot 2 | -- -- | -- | -- | -- |
| Metformin HCI | 87.5 kg | kg |  |  |
| Lactose USP 200 mesh | 26.25 kg | kg |  |  |
| Corn Starch | (8,750g) 8.75kg | kg |  |  |
| Plasdone K90 USP (PVP K90) | (1,050g) 1.05 kg | kg |  |  |
| Kollidon 90F | (1,750g) 1.75 kg | kg |  |  |
| Isopropyl Alcohol | (8L) 6.30 kg | kg |  |  |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |

________________	________________	Production Controller	Date of Issue of raw materials

Page 3.2

Blending/Granulation Record

Batch Size: 1,050,000’S

| Raw Materials | Theoretical Wt 350,000’s | Actual Weight 350,000’s | Signed & Date |  |
| --- | --- | --- | --- | --- |
|  |  |  | Received By | Checked By |
| Sub Lot 3 | -- -- | -- | -- | -- |
| Metformin HCI | 87.5 kg | kg |  |  |
| Lactose USP 200 mesh | 26.25 kg | kg |  |  |
| Corn Starch | (8,750g) 8.75kg | kg |  |  |
| Plasdone K90 USP (PVP K90) | (1,050g) 1.05 kg | kg |  |  |
| Kollidon 90F | (1,750g) 1.75 kg | kg |  |  |
| Isopropyl Alcohol | (8L) 6.30 kg | kg |  |  |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |
| -- | -- -- | -- | -- | -- |

________________	________________	Production Controller	Date of Issue of raw materials

Page 3.2

Blending/Granulation Record

Batch Size: 1,050,000’S

| Raw Materials | Theoretical Wt | Actual Weight | Signed & Date |  |
| --- | --- | --- | --- | --- |
|  |  |  | Received By | Checked By |
| -- | For 1050,000’ S | 1,050,000’ S | -- -- | -- -- |
| Magnesium Stearate | 1880.00 g | 1880.00 g |  |  |
| -- | -- -- | -- -- | -- -- | -- -- |
| -- | -- -- | -- -- | -- -- | -- -- |
| -- | -- -- | -- -- | -- -- | -- -- |
| -- | -- -- | -- -- |  |  |
| -- | -- -- | -- -- | -- -- | -- -- |
| -- | -- -- | -- -- | -- -- | -- -- |
| -- | -- -- | -- -- | -- -- | -- -- |
| -- | -- -- | -- -- |  |  |
| -- | -- -- | -- -- | -- -- | -- -- |
| -- | -- -- | -- -- | -- -- | -- -- |

________________	________________	Production Controller	Date of Issue of raw materials

Page 3.3

Page 3.4a

BINDING AGENT RECORD

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
| 1.0 | Binding agent preparation: (Sub Lot 1) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 1.1 | Take cleaned 15L SS container and place on electronic weighing balance, tare the weight of SS container, and pour the purified water from Jerri-can using measuring jar into SS container and weigh 6.0 kg (6L) of purified water. 取已洗净的15L不锈钢容器，称去容器的皮重，用量杯将罐中的纯化水倒入不锈钢容器中，称重6.0 kg（6L）纯化水。 |  |  |
| 1.2 | Take cleaned 30L SS container and take the tare weight of container and pour 6.30kg (8L) dispensed Isopropyl alcohol (IPA). Dip the power mixer blade shaft into the IPA contain container, start stirring to form vortex by adjusting variable speed button from 6 to 9 on power mixer and slowly add 1.05 kg of the Plasdone K90 USP (PVP K90) under stirring for about approx. 2 minute and continue the stirring for about 20 minutes. 取已洗净的30L SS容器，称取容器的皮重，倒入分配好的6.30kg (8L) Isopropyl alcohol (IPA)，将动力搅拌器叶片轴浸入IPA容器中，开始搅拌以形成涡流，方法是调节动力搅拌器上的变速按钮从 6到9，然后在搅拌下慢慢加入1.05 kg Plasdone K90 USP (PVP K90) 约2 分钟，继续搅拌约20分钟。 |  |  |
| 1.3 | Add the purified water from step no. 1.1 into step no. 1.2 and continue stirring for about 10 minutes. 将步骤1.1中的纯化水加入步骤1.2中，继续搅拌约10分钟。 Raw materials 原料; Std. Qty (kg) 标准数量; Actual weight (kg) 实际重量. Plasdone K90 (PVP K90); 1.05 kg. Isopropyl Alcohol; 6.30 kg (8L). Purified water; 6.0 kg (6L). Operator needs to check visually solution must be clear and lumps free and supervisor or controller shall verify physically solution condition for clear/lumps free solution. Filter the PVP solution through #40 mesh. 操作员需要目视检查溶液是否清澈、无结块，主管或控制员应物理验证溶液是否清澈、无结块。通过 #40 网过滤 PVP 溶液。 Binding agent solution condition (Clear & lumps free/not clear): _____________________________________________________ 粘合剂溶液状况（透明且无结块/不透明）:_____________________ Verified by qualified person sign & date: ____________________ 合格人员验证签名和日期： Note: If lumps observed on sieve take some filtered solution and pour on lumps and agitate with scrapper and allow to pass through the sieve. 注意：如果在筛子上发现块状物，则取一些过滤溶液倒在块状物上，并用刮刀搅拌，使其通过筛子。 After filtration of binding agent solution complete, check the weight and record in below table. 粘结剂溶液过滤完成后，测量重量并记录于下表中。 Tare weight of Container (kg)-A 容器皮重 (kg)-A; Gross weight (kg)-B 毛重 (kg)-B; Actual Weight of binding agent solution (kg)-(B-A) 粘合剂溶液实际重量 (kg)-(B-A). Note: The prepared PVP solution must be used within 24 hours. If solution store more than 1 hours stir the solution before usage. 注意：配制好的PVP溶液必须在24小时内使用。如果溶液存放时间超过1小时，使用前请搅拌溶液。 |  |  |

Page 3.3

Page 3.3

Page 3.4a

Blending/Granulation Procedure

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. |
| 2.0 | Milling: (Sub Lot 1) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 2.1 | Before starting of power mill, set the mill speed at medium speed and blade forward direction and record the details below, and circle on applicable parameters. 启动动力磨机前，将磨机速度设置为中速、刀片前进方向，并记录以下详细信息，并圈出适用的参数。 Mill speed: Slow/ Medium /Fast 研磨速度：慢/中/快 Blade direction: Forward / Reverse 刀片方向：前进/后退 Verified by qualified person sign & date: ________________ 合格人员验证签名和日期： |  |  |
| 2.2 | Prepare double polyethylene bag lined container and tie the polyethylene bag to the discharge hopper chute of power mill for collecting the milled material. 准备双层聚乙烯袋内衬容器，并将聚乙烯袋系在动力磨机的卸料料斗溜槽上，以收集研磨材料。 |  |  |

| 2.0 | Milling: (Sub Lot 1) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| --- | --- | --- | --- |
| 2.3 | De-lump the Metformin HCl before milling using hammer (knock on the Metformin HCl drum all around by using hammer). 研磨之前，先用锤子将Metformin HCl打碎（用锤子敲击Metformin HCl桶的四周）。 Pass the 87.50 kg Metformin HCl through power mill fixed with 2.0 mm screen at medium speed and blade forward direction and collect in tared double polyethylene bag lined container. 将 87.50 kg Metformin HCl以中等速度和刀片向前方向通过固定有 2.0 mm筛网的动力磨，并收集在双层聚乙烯袋内衬容器中。 |  |  |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. *Mention the details if the integrity of sieve/screen not satisfactory in the remarks column. |

Page 3.4a

| 3.0 | Sifting: (Sub Lot 1) | Page 3.4a Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| --- | --- | --- | --- |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. #100. #40. |
| 3.1 | Setup the mechanical sifter with cleaned #100 mesh and prepare tared double polyethylene bag lined container and attach the polyethylene bag to the discharge hopper chute by using rubber band for collecting the sieved material. 将清洁的#100筛子设置在机械筛上，并准备双层聚乙烯袋内衬容器，并使用橡皮筋将聚乙烯袋连接至卸料斗滑槽以收集筛分材料。 |  |  |
| 3.2 | Take the 8.75 kg of Corn Starch and verify the Lot no, Qty., QC reference no, and pass through #100 mesh (add corn starch scoop by scoop to the mechanical sifter) at the end of the sieving use Teflon scrapper to ensure all material passed through the sieve and collect the material in tared polyethylene bag lined container and keep aside. 取出8.75kg corn starch，核实批号、数量、QC参考号，再将其筛过#100目筛网（将玉米淀粉一勺一勺地加入到机械筛中），过筛结束时用特氟龙刮刀确保所有物料通过筛子，收集于内衬聚乙烯袋的容器中备用。 Note: Retained on # 100 mesh materials shall be collected and record the weight as process loss. 注意：应收集保留在#100目筛网上材料，并将其重量记录为过程损失。 |  |  |
| 3.3 | Setup the mechanical sifter with #40 mesh and prepare tared double polyethylene bag lined container and tie the polyethylene bag to the discharge hopper chute of mechanical sifter for collecting the sieved material. 将#40目筛网设置在机械筛上，准备双层聚乙烯袋内衬容器，将聚乙烯袋绑在机械筛的卸料斗斜槽上，用于收集筛分物。 |  |  |
| 3.4 | Take 1.75 kg Kollidon 90F and 26.25kg of Lactose and verify the Lot no, Qty., QC reference no and continue the sifting through #40 mesh and collect the materials in tared double polyethylene bag lined container. 取1.75 kg Kollidon 90F和26.25kg Lactose，验证批号、数量、QC参考号，并继续通过 #40 筛网过筛，并将材料收集在双层聚乙烯袋内衬容器中。 Note: Retained on # 40 mesh materials shall be collected and record the weight as process loss. 注意：应收集保留在#40目筛网上的材料，并将其重量记录为过程损失。 |  |  |
|  | Sieve size/screen 筛网尺寸; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. #100. #40. *Mention the details if the integrity of sieve/screen not satisfactory in the remarks column. |

Page 3.4a

Page 3.3

Page 3.4a

| 3.5 | Weigh and record the sieved materials in the below table. 称量筛分后的材料并记录在下表中。 Drum No. 鼓号; Tare weight (kg)-A (Weight of drum + polyethylene bag) 皮重 (kg) - A (桶 + 聚乙烯袋重量); Gross weight (kg)-B (Weight of material + A) 毛重 (kg)- B (材料重量+ A); Net weight (kg) (B-A) 净重 (kg) (B – A). Total weight (kg) 总重量 (kg). Weighed by sign & date: ________________ Checked by sign & date: ________________ 称重者签名和日期： 检查者签名和日期： |
| --- | --- |

Page 3.4a

Page 3.4a

Page 3.4b

DRY MIXING REPORT

| 4.0 | Dry mixing (Sub Lot 1) |
| --- | --- |
| 4.1 | Load approx. half quantity of milled Metformin HCl, sieved Corn Starch, sieved Lactose and sieved Kollidon 90F and remaining half quantity of milled metformin HCl in sequence into the Super Mixer and set the mixer set point 45Hz and chopper set point OFF, from touch panel screen. And dry mix for 300 ± 10 sec (290 sec – 310 sec) at mixer speed 45 Hz and chopper OFF. Record the mentioned parameters in below table. Mixer current shall be recorded at the end of dry mixing. 按照顺序将约一半量的研磨过的Metformin HCl、筛分过的Corn Starch、筛分过的Lactose和筛分过的 Kollidon 90F 以及剩余一半量的研磨过的Metformin HCl放入超级混合器中，并从触摸面板屏幕将混合器速度设定点设置为45Hz，将斩波器速度设置点设置为关闭，并在混合器速度45Hz和斩波器关闭下干混 300 ± 10 秒（290 秒 - 310 秒）。之后应在干混结束时记录下表中提到的混合器参数。 Dry mixing time 干混时间; Mixer Speed (Hz) 混合器速度; Mixer current (Amp) 混合器电流; Chopper speed (Hz) 斩波器 速度; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间(秒); Actual time (sec) 实际时间(秒); Set point 设定点; Actual point 实际点; Start开始; End 结束. (300±10 sec) Range: 290 – 310 sec (300±10秒) 范围： 290秒-310秒; 45 Hz; OFF 关闭. |

Page 3.4c

WET MIXING REPORT

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
| 5.0 | Wet Mixing: (Sub Lot 1) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 5.1 | Manually pour the binding agent solution of step no. 1.3 into the solution spray tank. 将步骤1.3的粘结剂溶液手动倒入溶液喷洒罐中。 |  |  |
| 5.2 | From the touch panel screen, touch on mixer speed “blue box” set the speed 10 Hz in keypad and touch “Enter” button. And touch on chopper “blue box” and set speed OFF in keypad and touch “Enter” button. (Refer to SOP Operation of Super Mixer (SWSG/PRD/0028)). 从触摸面板屏幕上，触摸混合器速度“蓝色框”，在键盘中将混合器速度设置为10Hz，然后触摸“Enter”按钮。然后触摸斩波器“蓝色框”并在键盘中设置斩波器速度为关闭，然后触摸“Enter”按钮（请参阅超级混合器的SOP操作 (SWSG/PRD/0028)）。 |  |  |
| 5.3 | Open the “Spray tank supply air” valve for compressed air supply to pump the binding agent solution and touch on “Mixer”, “Chopper” and “Spray” button to run the Super Mixer. 打开压缩空气供应的“喷雾罐供气”阀门，泵送粘结剂溶液，触摸“混合器”、“斩波器”和“喷雾”按钮以运行超级混合器。 Ensure spray tank supply air should be 2.0 -2.5 bar and record the observed value below. 确保喷雾罐供应空气应为2.0 -2.5bar ，并记录下面的观察值。 Spray tank supply air: __________________ 压缩空气气压： |  |  |

| Step | Manufacturing procedure |
| --- | --- |
| 5.4 | Add the binding agent solution using spray gun into the Super Mixer within 240 ± 10sec while mixing at mixer speed 10 Hz and Chopper speed OFF. Observe flow scale on solution spray tank become “0” to ensure all binding agent solution have been added into Super Mixer. Record the binding agent solution addition time and mixer current (amps) at the end of wet mixing. 使用喷枪将粘合剂溶液在240±10秒内添加到超级混合器中，同时以混合器速度10Hz和斩波器速度关闭混合。观察溶液喷雾罐上的流量刻度变为“0” ，以确保所有粘合剂溶液已添加到超级混合器中。记录湿式混合结束时的粘合剂溶液添加时间和混合器电流（Amp）。 Wet mixing (Binding agent addition) 粘合剂添加. Binding agent addition time (sec) 粘合剂添加时间(秒); Mixer Speed (Hz) 混合器速度; Chopper Speed (Hz) 斩波器速度; Mixer current (Amp) 混合器电流; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间(秒); Actual time (sec) 实际时间 (秒); Set Point 设定点; Actual Point 实际点; Set Point 设定点; Actual Point 实际点; Start 开始; End 结束. 240±10 sec Range: 230-250 sec 240±10秒; 10 Hz; OFF 关闭. |
| 5.5 | Perform the intermittent raking after binder addition. 添加粘合剂后进行间歇耙动。 Intermittent raking start time 间歇耙动开始时间; Intermittent raking end time 间歇耙草结束时间; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. |

Page 3.4c

| Step | Manufacturing procedure |
| --- | --- |
| 5.6 | Perform the kneading for 60 ± 10 sec at mixer speed 40Hz and Chopper speed at 45 Hz. Record the kneading time and mixer current (amps). 在混合器速度40Hz和斩波器速度45Hz下进行捏合60±10秒。记录捏合时间和搅拌器电流（Amp）。 Observe the granules condition by taking some quantity of wet mass and squeezing in hand, the lumps should be broken down to physically check the quality of granules, there should be no dry powder and fine powder. After checking the wet granules to be added back into Super Mixer. 观察颗粒情况，取适量湿团，用手挤压，将团块打碎，以物理检查颗粒的质量，不应有干粉和细粉。检查后将湿颗粒添加回超级混合器中。 Wet mixing 湿法混合（捏合）. Wet mixing time (sec) 捏合时间（秒）; Mixer Speed (Hz) 混合器速度; Chopper Speed (Hz) 斩波器速度; Mixer current (Amp) 混合器 电流; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间 (秒); Actual time (sec) 实际时间 (秒); Set Point 设定点; Actual Point 实际点; Set Point 设定点; Actual Point 实际点; Start开始; End 结束. 60 ±10sec Range: 50 – 70 sec 60±10 秒; 40 Hz; 45 Hz. |
| 5.7 | Place and hold the tared double polyethylene bag lined container at discharge chute and unload the wet granules by touching on “Discharge open” button in the control panel. 将双层聚乙烯袋内衬容器放置并保持在卸料槽处，然后通过触摸控制面板上的“卸料打开”按钮卸载湿颗粒。 Ensure that all the wet granules completely come out from Super Mixer. If require rake the Super Mixer using Teflon scraper or SS paddle to completely discharge the wet granules. 确保所有湿颗粒完全从超级混合器中出来。如果需要，使用聚四氟乙烯刮刀或不锈钢桨耙动超级混合器，以完全排出湿颗粒。 |

Page 3.4c

| Step | Manufacturing procedure |
| --- | --- |
| 5.8 | Weigh and divide step 5.7 wet granules into approx. 2 equal parts (FBD basin 1 and FBD basin 2). 称重并将步骤 5.7 的湿颗粒分成约2等份（FBD盆 1 和FBD盆 2）。 Drum No. 鼓号; Tare weight (kg) – A 皮重 (kg) - A; Gross weight (kg) – B 毛重 (kg) - B; Net weight (kg) – (B-A) 净重 (kg) - (B-A). Total weight (kg) 总重量 (kg). Part 1 (FBD basin 1) Qty.: ____________kg Part 2 (FBD basin 2) Qty.: ____________kg 第1部分 (FBD盆1) 数量：____________kg 第2部分 (FBD盆2) 数量：____________kg Weighed by sign & date: ________________ Checked by sign & date: __________________________ 称重者签名和日期：___________________检查者签名和日期：______ |

Page 3.4c

Page 3.4d

DRYING REPORT

| Step | Manufacturing procedure |
| --- | --- |
| 6.0 | Drying: (Sub Lot 1) |
| 6.1 | Before starting the drying process check the finger number and record the details below, 在开始干燥过程之前，检查手指的数量并记录以下详细信息， Finger bag no. 手指包号. Load the part 1 wet granules from step no. 5.8 into FBD basin 1 and push into Fluid Bed Dryer. Start the drying by setting Heater ON with inlet temperature at 60°C (Range: 60±10°C), switching ON the FBD Bowl stirrer, Bag shake interval timer is 120 sec, Bag shake duration timer is 15 sec. Refer the below table for blower speed, drying time and FBD bowl stirrer setting. 从步骤5.8中的第1部分湿颗粒放入FBD盆1并推入流化床干燥机，通过将加热器设置为ON（入口温度为60±10°C），打开FBD碗搅拌器，袋摇动间隔定时器为120秒，袋摇动持续时间定时器为15秒后开始干燥。 请参阅下表了解鼓风机速度、干燥时间和FBD碗搅拌器设置为干燥过程设置。 Perform the intermittent racking after 5 minutes of drying by taking out FBD basin, rack the materials bottom side granules up by using scoop or paddle and continue the drying process by switching ON the FBD bowl stirrer. 干燥5分钟后，取出FBD盆进行间歇式晾架，用勺子或桨将材料底部颗粒向上架起，然后打开FBD碗搅拌器继续干燥过程。 After complete drying process, collect pooled (5 different location) samples using spoon/scoop from FBD basic and send to IPQC for checking the LOD results using moisture analyser at 105°C temperature. If LOD results are found unsatisfactory, continue additional drying until desired LOD results are obtained. After drying process complete, record the LOD results in below table. LOD limit is 1.5 - 2.8% w/w (Target: 2.0%w/w). Page 3.4d 完成干燥过程后，使用勺在FBD盆收集合并的（5个不同位置）样品，并将其发送至IPQC，以便在105°C温度下使用水分分析仪检查LOD 结果。如果发现LOD结果不令人满意，请继续额外干燥，直至获得所需的LOD结果。干燥过程完成后，记录LOD结果在下表。LOD限值为1.5-2.8 % w/w（目标: 2.0 % w/w）。 |
|  | Drying Time (11 mins) 干燥时间 (11 分钟); Done By Sign & Date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. FBD Basin No. FBD盆编号; Drying time 干燥时间; Start time 开始时间; End time 结束时间; Blower鼓风机; FBD Bowl Stirrer FBD碗搅拌器; Inlet Temp. (°C) Range: 60±10°C 入口温度 (°C); Product temp. observed (°C) 产品温度 观测值 (NMT 40°C); Outlet temp. observed (°C) 出口温度 观测值 (NMT 40°C); Bag shake interval timer (sec) 袋子震动间隔计时器（秒）; Bag shake duration timer (sec) 袋子震动持续计时器（秒）; LOD Limit (1.5-2.8 %w/w) LOD限值. Set Point 设定点; Actual point 实际点; Set Point 设定点; Actual point 实际点. 1; 5 min 5分钟; 45 Hz; ON开; 60°C; 120; 15; NA. 6 min 6分钟; 40 Hz; 120; 15. Additional drying (if required). |
| 6.2 | Page 3.4d Load the part 2 wet granules from step no. 5.8 into FBD basin 2 and push into Fluid Bed Dryer and start the actual drying by setting Heater ON with inlet temperature at 60°C (range: 60±10°C), Bag shake interval timer is 120 sec, Bag shake duration timer is 15 sec. Refer the below table for blower speed, drying time and FBD bowl stirrer setting. 从步骤5.8中的第2部分湿颗粒放入FBD盆2并推入流化床干燥机，通过将加热器设置为ON（入口温度为60±10°C），打开FBD碗搅拌器，袋摇动间隔定时器为120秒，袋摇动持续时间定时器为15秒后开始干燥。 请参阅下表了解鼓风机速度、干燥时间和FBD碗搅拌器设置为干燥过程设置。 Perform the intermittent racking after 5 minutes of drying by taking out FBD basin, rack the materials bottom side granules up by using scoop or paddle and continue the drying process by switching ON the FBD bowl stirrer. 干燥5分钟后，取出FBD盆进行间歇式晾架，用勺子或桨将材料底部颗粒向上架起，然后打开FBD碗搅拌器继续干燥过程。 After complete drying process, collect pooled (5 different location) samples using spoon/scoop from FBD basic and send to IPQC for checking the LOD results using moisture analyser at 105°C temperature. If LOD results are found unsatisfactory, continue additional drying until desired LOD results are obtained. After drying process complete record, the LOD results in below table. LOD limit is 1.5 - 2.8% w/w. (Target: 2.0%w/w). 完成干燥过程后，使用勺在FBD盆收集合并的（5个不同位置）样品，并将其发送至IPQC，以便在105°C温度下使用水分分析仪检查LOD 结果。如果发现LOD结果不令人满意，请继续额外干燥，直至获得所需的LOD结果。干燥过程完成后，记录LOD结果在下表。LOD限值为1.5-2.8 % w/w（目标: 2.0 % w/w）。 Drying Time (11 mins) 干燥时间 (11 分钟); Done By Sign & Date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. FBD Basin No. FBD盆编号; Drying time 干燥时间; Start time 开始时间; End time 结束时间; Blower鼓风机; FBD Bowl Stirrer FBD碗搅拌器; Inlet Temp. (°C) Range: 60±10°C 入口温度 (°C); Product temp. observed (°C) 产品温度 观测值 (NMT 40°C); Outlet temp. observed (°C) 出口温度 观测值 (NMT 40°C); Bag shake interval timer (sec) 袋子震动间隔计时器（秒）; Bag shake duration timer (sec) 袋子震动持续计时器（秒）; LOD Limit (1.5-2.8 %w/w) LOD限值. Set Point 设定点; Actual point 实际点; Set Point 设定点; Actual point 实际点. 2; 5 min 5分钟; 45 Hz; ON开; 60°C; 120; 15; NA. 6 min 6分钟; 40 Hz; 120; 15. Additional drying (if required). |

Page 3.4d

Page 3.4e

MILLING REPORT

| 7.0 | Milling: (Sub Lot 1) |
| --- | --- |
| 7.1 | Note: Check the integrity of screen before and after milling activity and record the details in the below table. 注意：在铣削活动之前和之后检查屏幕的完整性，并将详细信息记录在下表中。 Screen size 屏幕; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. Before starting of power mill set the mill speed at medium speed and blade forward direction and record the details below, and circle on applicable parameters. 在启动动力研磨机之前，将研磨机速度设置为中速和刀片前进方向，并记录以下详细信息，并圈出适用的参 *Mill speed: Slow / Medium / Fast *研磨速度：慢 / 中 / 快 *Blade direction: Forward / Reverse *刀片方向：前进/ 后退 Verified by qualified person sign & date: _______________ 合格人员验证签名和日期： |
| 7.2 | Take tare weight of double polyethylene bag lined container, lid, cable tie and tie the polyethylene bag to the discharge hopper chute of power mill for collecting the milled material. 称取内衬双层聚乙烯袋的容器、盖子、扎带的皮重，并将聚乙烯袋系在动力磨机的卸料斗滑槽上，用于收集磨碎的物料。 |
| 7.3 | Pass the dried granules scoop by scoop through the power mill fixed with 2.0 mm screen at medium speed and blade forward direction. And collect the milled granules into double polyethylene bag lined container. 将干燥颗粒一勺一勺地以中速、刀片向前方向通过固定有2.0mm屏幕的动力磨。将磨碎的颗粒收集到内衬双层聚乙烯袋的容器中。 Note: Check the milled granules for any black particles generated due to friction of blades. DO NOT FEED EXCESS GRANULES AT ONE TIME 注意：检查研磨后的颗粒是否有因刀片摩擦而产生的黑色颗粒。不要一次喂食过量的颗粒。 Screen size 屏幕; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. *Mention the details if the integrity of sieve/screen not satisfactory in the remarks column. *如果屏幕的完整性不满意，请在备注栏中注明。 |
| 7.4 | Weigh and record the milled granules details in the table below. Ensure each blue drum should not exceed 25.0 kg of gross weight. And transfer the weighed blue drums to the intermediate room. 称重并记录下表中磨碎的颗粒的详细信息。确保每个蓝桶毛重不超过25.0公斤，并将已称重的蓝桶转移至中间房间。 Drum No. 鼓号; Tare weight (kg) - A (Weight of drum + polyethylene bag) 皮重 (kg) - A (桶 + 聚乙烯袋重量); Gross weight (kg) - B (Weight of material + A) 毛重 (kg) - B (材料重量 + A); Net weight (kg) (B-A) 净重(kg) (B – A). (Lot-1) Total weight (kg) 总重量 (kg). Weighed By sign & date: __________________ Checked by sign & date: _______________________ 称重者签名和日期：_________________检查者签名和日期：_______________________ Note: Repeat the same procedure from step no.1 to step 7.4 for remaining sub lots. 注意：对于剩余的子批次，重复从步骤1到步骤 7.4 的相同过程。 |

Page 3.4e

|  | In-process QC report shall be updated as below: Dried granules LOD results (page for pasting the printout) Batch no.; Sub lot no.; 1; FBD Basin no.:. LOD limit; 1.5 – 2.8 % w/w. Drying time: __________mins; Drying time: _______mins. |
| --- | --- |

Page 3.4f

|  | In-process QC report shall be updated as below: Dried granules LOD results (page for pasting the printout) Batch no.; Sub lot no.; 1; FBD Basin no.:. LOD limit; 1.5 – 2.8 % w/w. Drying time: __________mins; Drying time: _______mins. |
| --- | --- |

Page 3.4f

Page 3.3

Page 3.4a

BINDING AGENT RECORD

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
| 1.0 | Binding agent preparation: (Sub Lot 2) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 1.1 | Take cleaned 15L SS container and place on electronic weighing balance, tare the weight of SS container, and pour the purified water from Jerri-can using measuring jar into SS container and weigh 6.0 kg (6L) of purified water. 取已洗净的15L不锈钢容器，称去容器的皮重，用量杯将罐中的纯化水倒入不锈钢容器中，称重6.0 kg（6L）纯化水。 |  |  |
| 1.2 | Take cleaned 30L SS container and take the tare weight of container and pour 6.30kg (8L) dispensed Isopropyl alcohol (IPA), Dip the power mixer blade shaft into the IPA contain container start stirring to form vortex by adjusting variable speed button from 6 to 9 on power mixer and slowly add 1.05 kg of the Plasdone K90 USP (PVP K90) under stirring for about approx. 2 minute and continue the stirring for about 20 minutes. 取已洗净的30L SS容器，称取容器的皮重，倒入分配好的6.30kg (8L) Isopropyl alcohol (IPA)，将动力搅拌器叶片轴浸入IPA容器中，开始搅拌以形成涡流，方法是调节动力搅拌器上的变速按钮从 6到9，然后在搅拌下慢慢加入1.05 kg Plasdone K90 USP (PVP K90) 约2 分钟，继续搅拌约20分钟。 |  |  |
| 1.3 | Add the purified water from step no. 1.1 into step no. 1.2 and continue stirring for about 10 minutes. 将步骤1.1中的纯化水加入步骤1.2中，继续搅拌约10分钟。 Raw materials 原料; Std. Qty (kg) 标准数量; Actual weight (kg) 实际重量. Plasdone K90 (PVP K90); 1.05 kg. Isopropyl Alcohol; 6.30 kg (8L). Purified water; 6.0 kg (6L). Operator needs to check visually solution must be clear and lumps free and supervisor or controller shall verify physically solution condition for clear/lumps free solution. Filter the PVP solution through #40 mesh. 操作员需要目视检查溶液是否清澈、无结块，主管或控制员应物理验证溶液是否清澈、无结块。通过 #40 网过滤 PVP 溶液。 Binding agent solution condition (Clear & lumps free/not clear): _____________________________________________________ 粘合剂溶液状况（透明且无结块/不透明）:_____________________ Verified by qualified person sign & date: ____________________ 合格人员验证签名和日期： Note: If lumps observed on sieve take some filtered solution and pour on lumps and agitate with scrapper and allow to pass through the sieve. 注意：如果在筛子上发现块状物，则取一些过滤溶液倒在块状物上，并用刮刀搅拌，使其通过筛子。 After filtration of binding agent solution complete, check the weight and record in below table. 粘结剂溶液过滤完成后，测量重量并记录于下表中。 Tare weight of Container (kg)-A 容器皮重 (kg)-A; Gross weight (kg)-B 毛重 (kg)-B; Actual Weight of binding agent solution (kg)-(B-A) 粘合剂溶液实际重量 (kg)-(B-A). Note: The prepared PVP solution must be used within 24 hours. If solution store more than 1 hours stir the solution before usage. 注意：配制好的PVP溶液必须在24小时内使用。如果溶液存放时间超过1小时，使用前请搅拌溶液。 |  |  |

Page 3.3

Page 3.4a

Blending/Granulation Procedure

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. |
| 2.0 | Milling: (Sub Lot 2) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 2.1 | Before starting of power mill set the mill speed at medium speed and blade forward direction and record the details below, and circle on applicable parameters. 启动动力磨机前，将磨机速度设置为中速、刀片前进方向，并记录以下详细信息，并圈出适用的参数。 Mill speed: Slow/ Medium /Fast 研磨速度：慢/中/快 Blade direction: Forward / Reverse 刀片方向：前进/后退 Verified by qualified person sign & date: ________________ 合格人员验证签名和日期： |  |  |
| 2.2 | Prepare double polyethylene bag lined container and tie the polyethylene bag to the discharge hopper chute of power mill for collecting the milled material. 准备双层聚乙烯袋内衬容器，并将聚乙烯袋系在动力磨机的卸料料斗溜槽上，以收集研磨材料。 |  |  |

| 2.0 | Milling: (Sub Lot 2) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| --- | --- | --- | --- |
| 2.3 | De-lump the Metformin HCl before milling using hammer (knock on the Metformin HCl drum all around by using hammer). 研磨之前，先用锤子将Metformin HCl打碎（用锤子敲击Metformin HCl桶的四周）。 Pass the 87.50 kg Metformin HCl through power mill fixed with 2.0 mm screen at medium speed and blade forward direction and collect in double polyethylene bag lined container. 将 87.50 kg Metformin HCl以中等速度和刀片向前方向通过固定有 2.0 mm筛网的动力磨，并收集在双层聚乙烯袋内衬容器中。 |  |  |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. |

Page 3.4a

| 3.0 | Sifting: (Sub Lot 2) | Page 3.4a Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| --- | --- | --- | --- |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. #100. #40. |
| 3.1 | Setup the mechanical sifter with cleaned #100 mesh and prepare double polyethylene bag lined container and attach the polyethylene bag to the discharge hopper chute by using rubber band for collecting the sieved material. 将清洁的#100筛子设置在机械筛上，并准备双层聚乙烯袋内衬容器，并使用橡皮筋将聚乙烯袋连接至卸料斗滑槽以收集筛分材料。 |  |  |
| 3.2 | Take the 8.75 kg of corn starch and verify the Lot no, Qty., QC reference no, and pass through #100 mesh (add corn starch scoop by scoop to the mechanical sifter) at the end of the sieving use Teflon scrapper to ensure all material passed through the sieve and collect the material in polyethylene bag lined container and keep aside. 取出8.75kg corn starch，核实批号、数量、QC参考号，再将其筛过#100目筛网（将玉米淀粉一勺一勺地加入到机械筛中），过筛结束时用特氟龙刮刀确保所有物料通过筛子，收集于内衬聚乙烯袋的容器中备用。 Note: Retained on # 100 mesh materials shall be collected and record the weight as process loss. 注意：应收集保留在#100目筛网上材料，并将其重量记录为过程损失。 |  |  |
| 3.3 | Setup the mechanical sifter with #40 mesh and prepare double polyethylene bag lined container and tie the polyethylene bag to the discharge hopper chute of mechanical sifter for collecting the sieved material. 将40目筛网设置在机械筛上，准备双层聚乙烯袋内衬容器，将聚乙烯袋绑在机械筛的卸料斗斜槽上，用于收集筛分物。 |  |  |
| 3.4 | Take 1.75 kg Kollidon 90F and 26.25kg of Lactose and verify the Lot no, Qty., QC reference no and continue the sifting through #40 mesh and collect the materials in double polyethylene bag lined container. 取1.75 kg Kollidon 90F和26.25kg Lactose，验证批号、数量、QC参考号，并继续通过 #40 筛网过筛，并将材料收集在双层聚乙烯袋内衬容器中。 Note: Retained on # 40 mesh materials shall be collected and record the weight as process loss. 注意：应收集保留在#40目筛网上的材料，并将其重量记录为过程损失。 |  |  |
|  | Sieve size/screen 筛网尺寸; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. #100. #40. *Mention the details if the integrity of sieve/screen not satisfactory in the remarks column. |

Page 3.4a

Page 3.3

Page 3.4a

| 3.5 | Weigh and record the sieved materials in the below table. 称量筛分后的材料并记录在下表中。 Drum No. 鼓号; Tare weight (kg)-A (Weight of drum + polyethylene bag) 皮重 (kg) - A (桶 + 聚乙烯袋重量); Gross weight (kg)-B (Weight of material + A) 毛重 (kg)- B (材料重量+ A); Net weight (kg) (B-A) 净重 (kg) (B – A). Total weight (kg) 总重量 (kg). Weighed by sign & date: ________________ Checked by sign & date: ________________ 称重者签名和日期： 检查者签名和日期： |
| --- | --- |

Page 3.4a

Page 3.4a

Page 3.4b

DRY MIXING REPORT

| 4.0 | Dry mixing (Sub Lot 2) |
| --- | --- |
| 4.1 | Load approx. half quantity of milled metformin HCl, sieved corn starch, sieved lactose and sieved Kollidon 90F and remaining half quantity of milled metformin HCl in sequence into the Super Mixer and set the mixer set point 45Hz and chopper set point OFF, from touch panel screen. And dry mix for 300 ± 10 sec (290 sec – 310 sec] at mixer speed 45 Hz and chopper OFF. Record the mentioned parameters in below table. Mixer current shall be recorded at the end of dry mixing. 按照顺序将约一半量的研磨过的Metformin HCl、筛分过的corn starch、筛分过的lactose和筛分过的 Kollidon 90F 以及剩余一半量的研磨过的Metformin HCl放入超级混合器中，并从触摸面板屏幕将混合器速度设定点设置为45Hz，将斩波器速度设置点设置为关闭，并在混合器速度45Hz和斩波器关闭下干混 300 ± 10 秒（290 秒 - 310 秒）。之后应在干混结束时记录下表中提到的混合器参数。 Dry mixing time 干混时间; Mixer Speed (Hz) 混合器速度; Mixer current (Amp) 混合器电流; Chopper speed (Hz) 斩波器 速度; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间(秒); Actual time (sec) 实际时间(秒); Set point 设定点; Actual point 实际点; Start开始; End 结束. (300±10 sec) Range: 290 – 310 sec (300±10秒) 范围： 290秒-310秒; 45 Hz; OFF 关闭. |

Page 3.4c

WET MIXING REPORT

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
| 5.0 | Wet Mixing: (Sub Lot 2) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 5.1 | Manually pour the binding agent solution of step no. 1.3 into the solution spray tank. 将步骤1.3的粘结剂溶液手动倒入溶液喷洒罐中。 |  |  |
| 5.2 | From the touch panel screen, touch on mixer speed “blue box” set the speed 10 Hz in keypad and touch “Enter” button. And touch on chopper “blue box” and set speed OFF in keypad and touch “Enter” button. (Refer to SOP Operation of Super Mixer (SWSG/PRD/0028)). 从触摸面板屏幕上，触摸混合器速度“蓝色框”，在键盘中将混合器速度设置为10Hz，然后触摸“Enter”按钮。然后触摸斩波器“蓝色框”并在键盘中设置斩波器速度为关闭，然后触摸“Enter”按钮（请参阅超级混合器的SOP操作 (SWSG/PRD/0028)）。 |  |  |
| 5.3 | Open the “Spray tank supply air” valve for compressed air supply to pump the binding agent solution and touch on “Mixer”, “Chopper” and “Spray” button to run the super mixer. 打开压缩空气供应的“喷雾罐供气”阀门，泵送粘结剂溶液，触摸“混合器”、“斩波器”和“喷雾”按钮以运行超级混合器。 Ensure spray tank supply air should be 2.0 -2.5 bar and record the observed value below. 确保喷雾罐供应空气应为2.0 -2.5bar ，并记录下面的观察值。 Spray tank supply air: __________________ 压缩空气气压： |  |  |

| Step | Manufacturing procedure |
| --- | --- |
| 5.4 | Add the binding agent solution using spray gun into the super mixer within 240 ± 10sec while mixing at mixer speed 10 Hz and Chopper speed OFF. Observe flow scale on solution spray tank become “0” to ensure all binding agent solution have been added into super mixer. Record the binding agent solution addition time and mixer current (amps) at the end of wet mixing. 使用喷枪将粘合剂溶液在240±10秒内添加到超级混合器中，同时以混合器速度10Hz和斩波器速度关闭混合。观察溶液喷雾罐上的流量刻度变为“0”，以确保所有粘合剂溶液已添加到超级混合器中。记录湿式混合结束时的粘合剂溶液添加时间和混合器电流（Amp）。 Wet mixing (Binding agent addition) 粘合剂添加. Binding agent addition time (sec) 粘合剂添加时间(秒); Mixer Speed (Hz) 混合器速度; Chopper Speed (Hz) 斩波器速度; Mixer current (Amp) 混合器电流; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间(秒); Actual time (sec) 实际时间 (秒); Set Point 设定点; Actual Point 实际点; Set Point 设定点; Actual Point 实际点; Start 开始; End 结束. 240±10 sec 240±10秒; 10 Hz; OFF 关闭. |
| 5.5 | Perform the intermittent raking after binder addition. 添加粘合剂后进行间歇耙动。 Intermittent raking start time 间歇耙动开始时间; Intermittent raking end time 间歇耙草结束时间; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. |

Page 3.4c

| Step | Manufacturing procedure |
| --- | --- |
| 5.6 | Perform the kneading for 60 ± 10 sec at mixer speed 40Hz and Chopper speed at 45 Hz. Record the kneading time and mixer current (amps). 在混合器速度40Hz和斩波器速度45Hz下进行捏合60±10秒。记录捏合时间和搅拌器电流（Amp）。 Observe the granules condition by taking some quantity of wet mass and squeezing in hand, the lumps should be broken down to physically check the quality of granules, there should be no dry powder and fine powder. After checking the wet granules to be added back into super mixer. 观察颗粒情况，取适量湿团，用手挤压，将团块打碎，以物理检查颗粒的质量，不应有干粉和细粉。检查后将湿颗粒添加回超级混合器中。 Wet mixing 湿法混合（捏合）. Wet mixing time (sec) 捏合时间（秒）; Mixer Speed (Hz) 混合器速度; Chopper Speed (Hz) 斩波器速度; Mixer current (Amp) 混合器 电流; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间 (秒); Actual time (sec) 实际时间 (秒); Set Point 设定点; Actual Point 实际点; Set Point 设定点; Actual Point 实际点; Start开始; End 结束. 60 ±10sec 60±10 秒; 40 Hz; 45 Hz. |
| 5.7 | Place and hold the double polyethylene bag lined container at discharge chute and unload the wet granules by touching on “Discharge open” button in the control panel. 将双层聚乙烯袋内衬容器放置并保持在卸料槽处，然后通过触摸控制面板上的“卸料打开”按钮卸载湿颗粒。 Ensure that all the wet granules completely come out from super mixer. If require rake the super mixer using Teflon scraper or SS paddle to completely discharge the wet granules. 确保所有湿颗粒完全从超级混合器中出来。如果需要，使用聚四氟乙烯刮刀或不锈钢桨耙动超级混合器，以完全排出湿颗粒。 |

Page 3.4c

| Step | Manufacturing procedure |
| --- | --- |
| 5.8 | Weigh and divide step 5.7 wet granules into approx. 2 equal parts (FBD basin 1 and FBD basin 2). 称重并将步骤 5.7 的湿颗粒分成约2等份（FBD盆 1 和FBD盆 2）。 Drum No. 鼓号; Tare weight (kg) – A 皮重 (kg) - A; Gross weight (kg) – B 毛重 (kg) - B; Net weight (kg) – (B-A) 净重 (kg) - (B-A). Total weight (kg) 总重量 (kg). Part 1 (FBD basin 1) Qty.: ____________kg Part 2 (FBD basin 2) Qty.: ____________kg 第1部分 (FBD盆1) 数量：____________kg 第2部分 (FBD盆2) 数量：____________kg Weighed by sign & date: ________________ Checked by sign & date: __________________________ 称重者签名和日期：___________________检查者签名和日期：______ |

Page 3.4c

Page 3.4d

DRYING REPORT

| Step | Manufacturing procedure |
| --- | --- |
| 6.0 | Drying: (Sub Lot 2) |
| 6.1 | Before starting the drying process check the finger number and record the details below, 在开始干燥过程之前，检查手指的数量并记录以下详细信息， Finger bag no. 手指包号. Load the part 1 wet granules from step no. 5.8 into FBD basin 1 and push into Fluid bed dryer, Start the drying by setting Heater ON with inlet temperature at 60±10°C, switching ON the FBD Bowl stirrer, Bag shake interval timer is 120 sec, Bag shake duration timer is 15 sec. Refer the below table for blower speed, drying time and FBD bowl stirrer setting. 从步骤5.8中的第1部分湿颗粒放入FBD盆1并推入流化床干燥机，通过将加热器设置为ON（入口温度为60±10°C），打开FBD碗搅拌器，袋摇动间隔定时器为120秒，袋摇动持续时间定时器为15秒后开始干燥。 请参阅下表了解鼓风机速度、干燥时间和FBD碗搅拌器设置为干燥过程设置。 Perform the intermittent racking after 5 minutes of drying by taking out FBD basin, rack the materials bottom side granules up by using scoop or paddle and continue the drying process by switching ON the FBD Bowl stirrer. 干燥5分钟后，取出FBD盆进行间歇式晾架，用勺子或桨将材料底部颗粒向上架起，然后打开FBD碗搅拌器继续干燥过程。 Page 3.4d After complete drying process, collect pooled (5 different location) samples using spoon/scoop from FBD basic and send to IPQC for checking the LOD results using moisture analyser at 105°C temperature. If LOD results are found unsatisfactory, continue additional drying until desired LOD results are obtained. After drying process complete record, the LOD results in below table. LOD limit is 1.5 - 2.8% w/w (Target: 2.0%w/w). 完成干燥过程后，使用勺在FBD盆收集合并的（5个不同位置）样品，并将其发送至IPQC，以便在105°C温度下使用水分分析仪检查LOD 结果。如果发现LOD结果不令人满意，请继续额外干燥，直至获得所需的LOD结果。干燥过程完成后，记录LOD结果在下表。LOD限值为1.5-2.8 % w/w（目标: 2.0 % w/w）。 |
|  | Drying Time (11 mins) 干燥时间 (11 分钟); Done By Sign & Date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. FBD Basin No. FBD盆编号; Drying time 干燥时间; Start time 开始时间; End time 结束时间; Blower鼓风机; FBD Bowl Stirrer FBD碗搅拌器; Inlet Temp. (°C) 60±10°C 入口温度 (°C); Product temp. observed (°C) 产品温度 观测值 (NMT 40°C); Outlet temp. observed (°C) 出口温度 观测值 (NMT 40°C); Bag shake interval timer (sec) 袋子震动间隔计时器（秒）; Bag shake duration timer (sec) 袋子震动持续计时器（秒）; LOD Limit (1.5-2.8 %w/w) LOD限值. Set Point 设定点; Actual point 实际点; Set Point 设定点; Actual point 实际点. 1; 5 min 5分钟; 45 Hz; ON开; 60°C; 120; 15; NA. 6 min 6分钟; 40 Hz; 120; 15. Additional drying (if required). |
| 6.2 | Page 3.4d Load the part 2 wet granules from step no. 5.8 into FBD basin 2 and push into Fluid bed dryer and start the actual drying by setting Heater ON with inlet temperature at 60±10°C, Bag shake interval timer is 120 sec, Bag shake duration timer is 15 sec. Refer the below table for blower speed, drying time and FBD bowl stirrer setting. 从步骤5.8中的第2部分湿颗粒放入FBD盆2并推入流化床干燥机，通过将加热器设置为ON（入口温度为60±10°C），打开FBD碗搅拌器，袋摇动间隔定时器为120秒，袋摇动持续时间定时器为15秒后开始干燥。 请参阅下表了解鼓风机速度、干燥时间和FBD碗搅拌器设置为干燥过程设置。 Perform the intermittent racking after 5 minutes of drying by taking out FBD basin, rack the materials bottom side granules up by using scoop or paddle and continue the drying process by switching ON the FBD Bowl stirrer. 干燥5分钟后，取出FBD盆进行间歇式晾架，用勺子或桨将材料底部颗粒向上架起，然后打开FBD碗搅拌器继续干燥过程。 After complete drying process, collect pooled (5 different location) samples using spoon/scoop from FBD basic and send to IPQC for checking the LOD results using moisture analyser at 105°C temperature. If LOD results are found unsatisfactory, continue additional drying until desired LOD results are obtained. After drying process complete record, the LOD results in below table. LOD limit is 1.5 - 2.8% w/w. (Target: 2.0%w/w). 完成干燥过程后，使用勺在FBD盆收集合并的（5个不同位置）样品，并将其发送至IPQC，以便在105°C温度下使用水分分析仪检查LOD 结果。如果发现LOD结果不令人满意，请继续额外干燥，直至获得所需的LOD结果。干燥过程完成后，记录LOD结果在下表。LOD限值为1.5-2.8 % w/w（目标: 2.0 % w/w）。 Drying Time (11 mins) 干燥时间 (11 分钟); Done By Sign & Date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. FBD Basin No. FBD盆编号; Drying time 干燥时间; Start time 开始时间; End time 结束时间; Blower鼓风机; FBD Bowl Stirrer FBD碗搅拌器; Inlet Temp. (°C) 60±10°C 入口温度 (°C); Product temp. observed (°C) 产品温度 观测值 (NMT 40°C); Outlet temp. observed (°C) 出口温度 观测值 (NMT 40°C); Bag shake interval timer (sec) 袋子震动间隔计时器（秒）; Bag shake duration timer (sec) 袋子震动持续计时器（秒）; LOD Limit (1.5-2.8 %w/w) LOD限值. Set Point 设定点; Actual point 实际点; Set Point 设定点; Actual point 实际点. 2; 5 min 5分钟; 45 Hz; ON开; 60°C; 120; 15; NA. 6 min 6分钟; 40 Hz; 120; 15. Additional drying (if required). |

Page 3.4d

Page 3.4e

MILLING REPORT

| 7.0 | Milling: (Sub Lot 2) |
| --- | --- |
| 7.1 | Note: Check the integrity of screen before and after milling activity and record the details in the below table. 注意：在铣削活动之前和之后检查屏幕的完整性，并将详细信息记录在下表中。 Screen size 屏幕; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. Before starting of power mill set the mill speed at medium speed and blade forward direction and record the details below, and circle on applicable parameters. 在启动动力研磨机之前，将研磨机速度设置为中速和刀片前进方向，并记录以下详细信息，并圈出适用的参 *Mill speed: Slow / Medium / Fast *研磨速度：慢 / 中 / 快 *Blade direction: Forward / Reverse *刀片方向：前进/ 后退 Verified by qualified person sign & date: _______________ 合格人员验证签名和日期： |
| 7.2 | Take tare weight of double polyethylene bag lined container, Lid, cable tie and tie the polyethylene bag to the discharge hopper chute of power mill for collecting the milled material. 称取内衬双层聚乙烯袋的容器、盖子、扎带的皮重，并将聚乙烯袋系在动力磨机的卸料斗滑槽上，用于收集磨碎的物料。 |
| 7.3 | Pass the dried granules scoop by scoop through the power mill fixed with 2.0 mm screen at medium speed and blade forward direction. And collect the milled granules into double polyethylene bag lined container. 将干燥颗粒一勺一勺地以中速、刀片向前方向通过固定有2.0mm屏幕的动力磨。将磨碎的颗粒收集到内衬双层聚乙烯袋的容器中。 Note: Check the milled granules for any black particles generated due to friction of blades. DO NOT FEED EXCESS GRANULES AT ONE TIME 注意：检查研磨后的颗粒是否有因刀片摩擦而产生的黑色颗粒。不要一次喂食过量的颗粒。 Screen size 屏幕; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. *Mention the details if the integrity of sieve/screen not satisfactory in the remarks column. *如果屏幕的完整性不满意，请在备注栏中注明。 |
| 7.4 | Weigh and record the milled granules details in the table below. Ensure each blue drum should not exceed 25.0 kg of gross weight. And transfer the weighed blue drums to the intermediate room. 称重并记录下表中磨碎的颗粒的详细信息。确保每个蓝桶毛重不超过25.0公斤，并将已称重的蓝桶转移至中间房间。 Drum No. 鼓号; Tare weight (kg) - A (Weight of drum + polyethylene bag) 皮重 (kg) - A (桶 + 聚乙烯袋重量); Gross weight (kg) - B (Weight of material + A) 毛重 (kg) - B (材料重量 + A); Net weight (kg) (B-A) 净重(kg) (B – A). (Lot2) Total weight (kg) 总重量 (kg). Weighed By sign & date: __________________ Checked by sign & date: _______________________ 称重者签名和日期：_________________检查者签名和日期：_______________________ Note: Repeat the same procedure from step no.1 to step 7.4 for remaining sub lots. 注意：对于剩余的子批次，重复从步骤1到步骤 7.4 的相同过程。 |

Page 3.4e

Page 3.4e

|  | In-process QC report shall be updated as below: Dried granules LOD results (page for pasting the printout) Batch no.; Sub lot no.; 2; FBD Basin no.:. LOD limit; 1.5 – 2.8 % w/w. Drying time: __________mins; Drying time: _______mins. |
| --- | --- |

Page 3.4f

|  | In-process QC report shall be updated as below: Dried granules LOD results (page for pasting the printout) Batch no.; Sub lot no.; 2; FBD Basin no.:. LOD limit; 1.5 – 2.8 % w/w. Drying time: __________mins; Drying time: _______mins. |
| --- | --- |

Page 3.4f

Page 3.3

Page 3.4a

BINDING AGENT RECORD

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
| 1.0 | Binding agent preparation: (Sub Lot 3) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 1.1 | Take cleaned 15L SS container and place on electronic weighing balance, tare the weight of SS container, and pour the purified water from Jerri-can using measuring jar into SS container and weigh 6.0 kg (6L) of purified water. 取已洗净的15L不锈钢容器，称去容器的皮重，用量杯将罐中的纯化水倒入不锈钢容器中，称重6.0 kg（6L）纯化水。 |  |  |
| 1.2 | Take cleaned 30L SS container and take the tare weight of container and pour 6.30kg (8L) dispensed Isopropyl alcohol (IPA), Dip the power mixer blade shaft into the IPA contain container start stirring to form vortex by adjusting variable speed button from 6 to 9 on power mixer and slowly add 1.05 kg of the Plasdone K90 USP (PVP K90) under stirring for about approx. 2 minute and continue the stirring for about 20 minutes. 取已洗净的30L SS容器，称取容器的皮重，倒入分配好的6.30kg (8L) Isopropyl alcohol (IPA)，将动力搅拌器叶片轴浸入IPA容器中，开始搅拌以形成涡流，方法是调节动力搅拌器上的变速按钮从 6到9，然后在搅拌下慢慢加入1.05 kg Plasdone K90 USP (PVP K90) 约2 分钟，继续搅拌约20分钟。 |  |  |
| 1.3 | Add the purified water from step no. 1.1 into step no. 1.2 and continue stirring for about 10 minutes. 将步骤1.1中的纯化水加入步骤1.2中，继续搅拌约10分钟。 Raw materials 原料; Std. Qty (kg) 标准数量; Actual weight (kg) 实际重量. Plasdone K90 (PVP K90); 1.05 kg. Isopropyl Alcohol; 6.30 kg (8L). Purified water; 6.0 kg (6L). Operator needs to check visually solution must be clear and lumps free and supervisor or controller shall verify physically solution condition for clear/lumps free solution. Filter the PVP solution through #40 mesh. 操作员需要目视检查溶液是否清澈、无结块，主管或控制员应物理验证溶液是否清澈、无结块。通过 #40 网过滤 PVP 溶液。 Binding agent solution condition (Clear & lumps free/not clear): _____________________________________________________ 粘合剂溶液状况（透明且无结块/不透明）:_____________________ Verified by qualified person sign & date: ____________________ 合格人员验证签名和日期： Note: If lumps observed on sieve take some filtered solution and pour on lumps and agitate with scrapper and allow to pass through the sieve. 注意：如果在筛子上发现块状物，则取一些过滤溶液倒在块状物上，并用刮刀搅拌，使其通过筛子。 After filtration of binding agent solution complete, check the weight and record in below table. 粘结剂溶液过滤完成后，测量重量并记录于下表中。 Tare weight of Container (kg)-A 容器皮重 (kg)-A; Gross weight (kg)-B 毛重 (kg)-B; Actual Weight of binding agent solution (kg)-(B-A) 粘合剂溶液实际重量 (kg)-(B-A). Note: The prepared PVP solution must be used within 24 hours. If solution store more than 1 hours stir the solution before usage. 注意：配制好的PVP溶液必须在24小时内使用。如果溶液存放时间超过1小时，使用前请搅拌溶液。 |  |  |

Page 3.3

Page 3.4a

Blending/Granulation Procedure

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. |
| 2.0 | Milling: (Sub Lot 3) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 2.1 | Before starting of power mill set the mill speed at medium speed and blade forward direction and record the details below, and circle on applicable parameters. 启动动力磨机前，将磨机速度设置为中速、刀片前进方向，并记录以下详细信息，并圈出适用的参数。 Mill speed: Slow/ Medium /Fast 研磨速度：慢/中/快 Blade direction: Forward / Reverse 刀片方向：前进/后退 Verified by qualified person sign & date: ________________ 合格人员验证签名和日期： |  |  |
| 2.2 | Prepare double polyethylene bag lined container and tie the polyethylene bag to the discharge hopper chute of power mill for collecting the milled material. 准备双层聚乙烯袋内衬容器，并将聚乙烯袋系在动力磨机的卸料料斗溜槽上，以收集研磨材料。 |  |  |

| 2.0 | Milling: (Sub Lot 3) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| --- | --- | --- | --- |
| 2.3 | De-lump the Metformin HCl before milling using hammer (knock on the Metformin HCl drum all around by using hammer). 研磨之前，先用锤子将Metformin HCl打碎（用锤子敲击Metformin HCl桶的四周）。 Pass the 87.50 kg Metformin HCl through power mill fixed with 2.0 mm screen at medium speed and blade forward direction and collect in double polyethylene bag lined container. 将 87.50 kg Metformin HCl以中等速度和刀片向前方向通过固定有 2.0 mm筛网的动力磨，并收集在双层聚乙烯袋内衬容器中。 |  |  |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. |

Page 3.4a

| 3.0 | Sifting: (Sub Lot 3) | Page 3.4a Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| --- | --- | --- | --- |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. 注意：在筛分活动之前和之后检查筛网的完整性，并将详细信息记录在下表中。 Sieve size/screen 筛网尺寸; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. #100. #40. |
| 3.1 | Setup the mechanical sifter with cleaned #100 mesh and prepare double polyethylene bag lined container and attach the polyethylene bag to the discharge hopper chute by using rubber band for collecting the sieved material. 将清洁的#100筛子设置在机械筛上，并准备双层聚乙烯袋内衬容器，并使用橡皮筋将聚乙烯袋连接至卸料斗滑槽以收集筛分材料。 |  |  |
| 3.2 | Take the 8.75 kg of corn starch and verify the Lot no, Qty., QC reference no, and pass through #100 mesh (add corn starch scoop by scoop to the mechanical sifter) at the end of the sieving use Teflon scrapper to ensure all material passed through the sieve and collect the material in polyethylene bag lined container and keep aside. 取出8.75kg corn starch，核实批号、数量、QC参考号，再将其筛过#100目筛网（将玉米淀粉一勺一勺地加入到机械筛中），过筛结束时用特氟龙刮刀确保所有物料通过筛子，收集于内衬聚乙烯袋的容器中备用。 Note: Retained on # 100 mesh materials shall be collected and record the weight as process loss. 注意：应收集保留在#100目筛网上材料，并将其重量记录为过程损失。 |  |  |
| 3.3 | Setup the mechanical sifter with #40 mesh and prepare double polyethylene bag lined container and tie the polyethylene bag to the discharge hopper chute of mechanical sifter for collecting the sieved material. 将40目筛网设置在机械筛上，准备双层聚乙烯袋内衬容器，将聚乙烯袋绑在机械筛的卸料斗斜槽上，用于收集筛分物。 |  |  |
| 3.4 | Take 1.75 kg Kollidon 90F and 26.25kg of Lactose and verify the Lot no, Qty., QC reference no and continue the sifting through #40 mesh and collect the materials in double polyethylene bag lined container. 取1.75 kg Kollidon 90F和26.25kg Lactose，验证批号、数量、QC参考号，并继续通过 #40 筛网过筛，并将材料收集在双层聚乙烯袋内衬容器中。 Note: Retained on # 40 mesh materials shall be collected and record the weight as process loss. 注意：应收集保留在#40目筛网上的材料，并将其重量记录为过程损失。 |  |  |
|  | Sieve size/screen 筛网尺寸; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. #100. #40. *Mention the details if the integrity of sieve/screen not satisfactory in the remarks column. |

Page 3.4a

Page 3.3

Page 3.4a

| 3.5 | Weigh and record the sieved materials in the below table. 称量筛分后的材料并记录在下表中。 Drum No. 鼓号; Tare weight (kg)-A (Weight of drum + polyethylene bag) 皮重 (kg) - A (桶 + 聚乙烯袋重量); Gross weight (kg)-B (Weight of material + A) 毛重 (kg)- B (材料重量+ A); Net weight (kg) (B-A) 净重 (kg) (B – A). Total weight (kg) 总重量 (kg). Weighed by sign & date: ________________ Checked by sign & date: ________________ 称重者签名和日期： 检查者签名和日期： |
| --- | --- |

Page 3.4a

Page 3.4a

Page 3.4b

DRY MIXING REPORT

| 4.0 | Dry mixing (Sub Lot 3) |
| --- | --- |
| 4.1 | Load approx. half quantity of milled metformin HCl, sieved corn starch, sieved lactose and sieved Kollidon 90F and remaining half quantity of milled metformin HCl in sequence into the Super Mixer and set the mixer set point 45Hz and chopper set point OFF, from touch panel screen. And dry mix for 300 ± 10 sec (290 sec – 310 sec] at mixer speed 45 Hz and chopper OFF. Record the mentioned parameters in below table. Mixer current shall be recorded at the end of dry mixing. 按照顺序将约一半量的研磨过的Metformin HCl、筛分过的corn starch、筛分过的lactose和筛分过的 Kollidon 90F 以及剩余一半量的研磨过的Metformin HCl放入超级混合器中，并从触摸面板屏幕将混合器速度设定点设置为45Hz，将斩波器速度设置点设置为关闭，并在混合器速度45Hz和斩波器关闭下干混 300 ± 10 秒（290 秒 - 310 秒）。之后应在干混结束时记录下表中提到的混合器参数。 Dry mixing time 干混时间; Mixer Speed (Hz) 混合器速度; Mixer current (Amp) 混合器电流; Chopper speed (Hz) 斩波器 速度; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间(秒); Actual time (sec) 实际时间(秒); Set point 设定点; Actual point 实际点; Start开始; End 结束. (300±10 sec) Range: 290 – 310 sec (300±10秒) 范围： 290秒-310秒; 45 Hz; OFF 关闭. |

Page 3.4c

WET MIXING REPORT

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
| 5.0 | Wet Mixing: (Sub Lot 3) | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 5.1 | Manually pour the binding agent solution of step no. 1.3 into the solution spray tank. 将步骤1.3的粘结剂溶液手动倒入溶液喷洒罐中。 |  |  |
| 5.2 | From the touch panel screen, touch on mixer speed “blue box” set the speed 10 Hz in keypad and touch “Enter” button. And touch on chopper “blue box” and set speed OFF in keypad and touch “Enter” button. (Refer to SOP Operation of Super Mixer (SWSG/PRD/0028)). 从触摸面板屏幕上，触摸混合器速度“蓝色框”，在键盘中将混合器速度设置为10Hz，然后触摸“Enter”按钮。然后触摸斩波器“蓝色框”并在键盘中设置斩波器速度为关闭，然后触摸“Enter”按钮（请参阅超级混合器的SOP操作 (SWSG/PRD/0028)）。 |  |  |
| 5.3 | Open the “Spray tank supply air” valve for compressed air supply to pump the binding agent solution and touch on “Mixer”, “Chopper” and “Spray” button to run the super mixer. 打开压缩空气供应的“喷雾罐供气”阀门，泵送粘结剂溶液，触摸“混合器”、“斩波器”和“喷雾”按钮以运行超级混合器。 Ensure spray tank supply air should be 2.0 -2.5 bar and record the observed value below. 确保喷雾罐供应空气应为2.0 -2.5bar ，并记录下面的观察值。 Spray tank supply air: __________________ 压缩空气气压： |  |  |

| Step | Manufacturing procedure |
| --- | --- |
| 5.4 | Add the binding agent solution using spray gun into the super mixer within 240 ± 10sec while mixing at mixer speed 10 Hz and Chopper speed OFF. Observe flow scale on solution spray tank become “0” to ensure all binding agent solution have been added into super mixer. Record the binding agent solution addition time and mixer current (amps) at the end of wet mixing. 使用喷枪将粘合剂溶液在240±10秒内添加到超级混合器中，同时以混合器速度10Hz和斩波器速度关闭混合。观察溶液喷雾罐上的流量刻度变为“0”，以确保所有粘合剂溶液已添加到超级混合器中。记录湿式混合结束时的粘合剂溶液添加时间和混合器电流（Amp）。 Wet mixing (Binding agent addition) 粘合剂添加. Binding agent addition time (sec) 粘合剂添加时间(秒); Mixer Speed (Hz) 混合器速度; Chopper Speed (Hz) 斩波器速度; Mixer current (Amp) 混合器电流; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间(秒); Actual time (sec) 实际时间 (秒); Set Point 设定点; Actual Point 实际点; Set Point 设定点; Actual Point 实际点; Start 开始; End 结束. 240±10 sec 240±10秒; 10 Hz; OFF 关闭. |
| 5.5 | Perform the intermittent raking after binder addition. 添加粘合剂后进行间歇耙动。 Intermittent raking start time 间歇耙动开始时间; Intermittent raking end time 间歇耙草结束时间; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. |

Page 3.4c

| Step | Manufacturing procedure |
| --- | --- |
| 5.6 | Perform the kneading for 60 ± 10 sec at mixer speed 40Hz and Chopper speed at 45 Hz. Record the kneading time and mixer current (amps). 在混合器速度40Hz和斩波器速度45Hz下进行捏合60±10秒。记录捏合时间和搅拌器电流（Amp）。 Observe the granules condition by taking some quantity of wet mass and squeezing in hand, the lumps should be broken down to physically check the quality of granules, there should be no dry powder and fine powder. After checking the wet granules to be added back into super mixer. 观察颗粒情况，取适量湿团，用手挤压，将团块打碎，以物理检查颗粒的质量，不应有干粉和细粉。检查后将湿颗粒添加回超级混合器中。 Wet mixing 湿法混合（捏合）. Wet mixing time (sec) 捏合时间（秒）; Mixer Speed (Hz) 混合器速度; Chopper Speed (Hz) 斩波器速度; Mixer current (Amp) 混合器 电流; Done by sign & date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. Set time (sec) 设定时间 (秒); Actual time (sec) 实际时间 (秒); Set Point 设定点; Actual Point 实际点; Set Point 设定点; Actual Point 实际点; Start开始; End 结束. 60 ±10sec 60±10 秒; 40 Hz; 45 Hz. |
| 5.7 | Place and hold the double polyethylene bag lined container at discharge chute and unload the wet granules by touching on “Discharge open” button in the control panel. 将双层聚乙烯袋内衬容器放置并保持在卸料槽处，然后通过触摸控制面板上的“卸料打开”按钮卸载湿颗粒。 Ensure that all the wet granules completely come out from super mixer. If require rake the super mixer using Teflon scraper or SS paddle to completely discharge the wet granules. 确保所有湿颗粒完全从超级混合器中出来。如果需要，使用聚四氟乙烯刮刀或不锈钢桨耙动超级混合器，以完全排出湿颗粒。 |

Page 3.4c

| Step | Manufacturing procedure |
| --- | --- |
| 5.8 | Weigh and divide step 5.7 wet granules into approx. 2 equal parts (FBD basin 1 and FBD basin 2). 称重并将步骤 5.7 的湿颗粒分成约2等份（FBD盆 1 和FBD盆 2）。 Drum No. 鼓号; Tare weight (kg) – A 皮重 (kg) - A; Gross weight (kg) – B 毛重 (kg) - B; Net weight (kg) – (B-A) 净重 (kg) - (B-A). Total weight (kg) 总重量 (kg). Part 1 (FBD basin 1) Qty.: ____________kg Part 2 (FBD basin 2) Qty.: ____________kg 第1部分 (FBD盆1) 数量：____________kg 第2部分 (FBD盆2) 数量：____________kg Weighed by sign & date: ________________ Checked by sign & date: __________________________ 称重者签名和日期：___________________检查者签名和日期：______ |

Page 3.4c

Page 3.4d

DRYING REPORT

| Step | Manufacturing procedure |
| --- | --- |
| 6.0 | Drying: (Sub Lot 3) |
| 6.1 | Before starting the drying process check the finger number and record the details below, 在开始干燥过程之前，检查手指的数量并记录以下详细信息， Finger bag no. 手指包号. Load the part 1 wet granules from step no. 5.8 into FBD basin 1 and push into Fluid bed dryer, Start the drying by setting Heater ON with inlet temperature at 60±10°C, switching ON the FBD Bowl stirrer, Bag shake interval timer is 120 sec, Bag shake duration timer is 15 sec. Refer the below table for blower speed, drying time and FBD bowl stirrer setting. 从步骤5.8中的第1部分湿颗粒放入FBD盆1并推入流化床干燥机，通过将加热器设置为ON（入口温度为60±10°C），打开FBD碗搅拌器，袋摇动间隔定时器为120秒，袋摇动持续时间定时器为15秒后开始干燥。 请参阅下表了解鼓风机速度、干燥时间和FBD碗搅拌器设置为干燥过程设置。 Perform the intermittent racking after 5 minutes of drying by taking out FBD basin, rack the materials bottom side granules up by using scoop or paddle and continue the drying process by switching ON the FBD Bowl stirrer. 干燥5分钟后，取出FBD盆进行间歇式晾架，用勺子或桨将材料底部颗粒向上架起，然后打开FBD碗搅拌器继续干燥过程。 Page 3.4d After complete drying process, collect pooled (5 different location) samples using spoon/scoop from FBD basic and send to IPQC for checking the LOD results using moisture analyser at 105°C temperature. If LOD results are found unsatisfactory, continue additional drying until desired LOD results are obtained. After drying process complete record, the LOD results in below table. LOD limit is 1.5 - 2.8% w/w (Target: 2.0%w/w). Page 3.4d 完成干燥过程后，使用勺在FBD盆收集合并的（5个不同位置）样品，并将其发送至IPQC，以便在105°C温度下使用水分分析仪检查LOD 结果。如果发现LOD结果不令人满意，请继续额外干燥，直至获得所需的LOD结果。干燥过程完成后，记录LOD结果在下表。LOD限值为1.5-2.8 % w/w（目标: 2.0 % w/w）。 |
|  | Drying Time (11 mins) 干燥时间 (11 分钟); Done By Sign & Date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. FBD Basin No. FBD盆编号; Drying time 干燥时间; Start time 开始时间; End time 结束时间; Blower鼓风机; FBD Bowl Stirrer FBD碗搅拌器; Inlet Temp. (°C) 60±10°C 入口温度 (°C); Product temp. observed (°C) 产品温度 观测值 (NMT 40°C); Outlet temp. observed (°C) 出口温度 观测值 (NMT 40°C); Bag shake interval timer (sec) 袋子震动间隔计时器（秒）; Bag shake duration timer (sec) 袋子震动持续计时器（秒）; LOD Limit (1.5-2.8 %w/w) LOD限值. Set Point 设定点; Actual point 实际点; Set Point 设定点; Actual point 实际点. 1; 5 min 5分钟; 45 Hz; ON开; 60°C; 120; 15; NA. 6 min 6分钟; 40 Hz; 120; 15. Additional drying (if required). |
| 6.2 | Page 3.4d Load the part 2 wet granules from step no. 5.8 into FBD basin 2 and push into Fluid bed dryer and start the actual drying by setting Heater ON with inlet temperature at 60±10°C, Bag shake interval timer is 120 sec, Bag shake duration timer is 15 sec. Refer the below table for blower speed, drying time and FBD bowl stirrer setting. 从步骤5.8中的第2部分湿颗粒放入FBD盆2并推入流化床干燥机，通过将加热器设置为ON（入口温度为60±10°C），打开FBD碗搅拌器，袋摇动间隔定时器为120秒，袋摇动持续时间定时器为15秒后开始干燥。 请参阅下表了解鼓风机速度、干燥时间和FBD碗搅拌器设置为干燥过程设置。 Perform the intermittent racking after 5 minutes of drying by taking out FBD basin, rack the materials bottom side granules up by using scoop or paddle and continue the drying process by switching ON the FBD Bowl stirrer. 干燥5分钟后，取出FBD盆进行间歇式晾架，用勺子或桨将材料底部颗粒向上架起，然后打开FBD碗搅拌器继续干燥过程。 After complete drying process, collect pooled (5 different location) samples using spoon/scoop from FBD basic and send to IPQC for checking the LOD results using moisture analyser at 105°C temperature. If LOD results are found unsatisfactory, continue additional drying until desired LOD results are obtained. After drying process complete record, the LOD results in below table. LOD limit is 1.5 - 2.8% w/w. (Target: 2.0%w/w). 完成干燥过程后，使用勺在FBD盆收集合并的（5个不同位置）样品，并将其发送至IPQC，以便在105°C温度下使用水分分析仪检查LOD 结果。如果发现LOD结果不令人满意，请继续额外干燥，直至获得所需的LOD结果。干燥过程完成后，记录LOD结果在下表。LOD限值为1.5-2.8 % w/w（目标: 2.0 % w/w）。 Drying Time (11 mins) 干燥时间 (11 分钟); Done By Sign & Date 完成者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. FBD Basin No. FBD盆编号; Drying time 干燥时间; Start time 开始时间; End time 结束时间; Blower鼓风机; FBD Bowl Stirrer FBD碗搅拌器; Inlet Temp. (°C) 60±10°C 入口温度 (°C); Product temp. observed (°C) 产品温度 观测值 (NMT 40°C); Outlet temp. observed (°C) 出口温度 观测值 (NMT 40°C); Bag shake interval timer (sec) 袋子震动间隔计时器（秒）; Bag shake duration timer (sec) 袋子震动持续计时器（秒）; LOD Limit (1.5-2.8 %w/w) LOD限值. Set Point 设定点; Actual point 实际点; Set Point 设定点; Actual point 实际点. 2; 5 min 5分钟; 45 Hz; ON开; 60°C; 120; 15; NA. 6 min 6分钟; 40 Hz; 120; 15. Additional drying (if required). |

Page 3.4d

Page 3.4e

MILLING REPORT

| 7.0 | Milling: (Sub Lot 3) |
| --- | --- |
| 7.1 | Note: Check the integrity of screen before and after milling activity and record the details in the below table. 注意：在铣削活动之前和之后检查屏幕的完整性，并将详细信息记录在下表中。 Screen size 屏幕; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. Before starting of power mill set the mill speed at medium speed and blade forward direction and record the details below, and circle on applicable parameters. 在启动动力研磨机之前，将研磨机速度设置为中速和刀片前进方向，并记录以下详细信息，并圈出适用的参 *Mill speed: Slow / Medium / Fast *研磨速度：慢 / 中 / 快 *Blade direction: Forward / Reverse *刀片方向：前进/ 后退 Verified by qualified person sign & date: _______________ 合格人员验证签名和日期： |
| 7.2 | Take tare weight of double polyethylene bag lined container, Lid, cable tie and tie the polyethylene bag to the discharge hopper chute of power mill for collecting the milled material. 称取内衬双层聚乙烯袋的容器、盖子、扎带的皮重，并将聚乙烯袋系在动力磨机的卸料斗滑槽上，用于收集磨碎的物料。 |
| 7.3 | Pass the dried granules scoop by scoop through the power mill fixed with 2.0 mm screen at medium speed and blade forward direction. And collect the milled granules into double polyethylene bag lined container. 将干燥颗粒一勺一勺地以中速、刀片向前方向通过固定有2.0mm屏幕的动力磨。将磨碎的颗粒收集到内衬双层聚乙烯袋的容器中。 Note: Check the milled granules for any black particles generated due to friction of blades. DO NOT FEED EXCESS GRANULES AT ONE TIME 注意：检查研磨后的颗粒是否有因刀片摩擦而产生的黑色颗粒。不要一次喂食过量的颗粒。 Screen size 屏幕; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. 2.0 mm. *Mention the details if the integrity of sieve/screen not satisfactory in the remarks column. *如果屏幕的完整性不满意，请在备注栏中注明。 |
| 7.4 | Weigh and record the milled granules details in the table below. Ensure each blue drum should not exceed 25.0 kg of gross weight. And transfer the weighed blue drums to the intermediate room. 称重并记录下表中磨碎的颗粒的详细信息。确保每个蓝桶毛重不超过25.0公斤，并将已称重的蓝桶转移至中间房间。 Drum No. 鼓号; Tare weight (kg) - A (Weight of drum + polyethylene bag) 皮重 (kg) - A (桶 + 聚乙烯袋重量); Gross weight (kg) - B (Weight of material + A) 毛重 (kg) - B (材料重量 + A); Net weight (kg) (B-A) 净重(kg) (B – A). (Lot 3) Total weight (kg) 总重量 (kg). Weighed By sign & date: __________________ Checked by sign & date: _______________________ 称重者签名和日期：_________________检查者签名和日期：_______________________ Note: Repeat the same procedure from step no.1 to step 7.4 for remaining sub lots. 注意：对于剩余的子批次，重复从步骤1到步骤 7.4 的相同过程。 |

Page 3.4e

|  | In-process QC report shall be updated as below: Dried granules LOD results (page for pasting the printout) Batch no.; Sub lot no.; 3; FBD Basin no.:. LOD limit; 1.5 – 2.8 % w/w. Drying time: __________mins; Drying time: _______mins. |
| --- | --- |

Page 3.4f

|  | In-process QC report shall be updated as below: Dried granules LOD results (page for pasting the printout) Batch no.; Sub lot no.; 3; FBD Basin no.:. LOD limit; 1.5 – 2.8 % w/w. Drying time: __________mins; Drying time: _______mins. |
| --- | --- |

Page 3.4f

| Page 3.3 Page 3.4e Page 3.3 Page 3.4d Page 3.4e Page 3.4g | To be completed by Production supervisor: Yield and Reconciliation. A; Theoretical Yield (Theoretical weight of blend); 375.90 kg. B; Actual weight of milled dried granules (Lot-1+Lot-2+Lot-3); __________kg. C; Percentage of Theoretical yield (B/A x 100); __________%. *Yield limit is 95.0 – 102 % Calculated by sign & date: _______________ Checked by sign & date: __________________ |
| --- | --- |

Page 3.4h

In-Process QC Report

| MOISTURE SAMPLE (COLLECTED BEFORE MILLING PROCESS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Moisture sample NMT 4.0% (Sunward Method) | Sub lot | FBD Bowl No. | Approximate Quantity (g) | Sign & Date | Time (am/pm) | Result |
|  | 1 | 1 |  |  |  |  |
|  |  | 2 |  |  |  |  |
|  | 2 | 1 |  |  |  |  |
|  |  | 2 |  |  |  |  |
|  | 3 | 1 |  |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  |  |  |  | Average Moisture (%) |  |

Result: Passed / Failed

Remarks:               _______________________________________________________________________________

_______________________________________________________________________________

_____________________	_____________________

Analysed By	Checked By

_____________________	_____________________

Date	Date

Page 3.4i

Blending Area Checklist

| GMP Checklist: |  |  |  | Date: | Time: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BLENDING DEPARTMENT |  |  | Tick(✓) where appropriate |  | Sign & Date By |  |  |
|  |  |  |  |  | Done by |  | Production Supervisor |
| Work Area Check | Materials from previous batch are removed |  | □ No | □ Yes |  |  |  |
|  | Room name: |  | □ Unclean | □ Clean |  |  |  |
|  | Room is labelled |  | □ No | □ Yes |  |  |  |
|  | Status of equipment |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Vacuum Cleaner |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Multi-Directional Mixer |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Exhaust |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Air-con |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Weighing Balance |  | □ Unclean | □ Clean |  |  |  |
|  | All equipment are labelled |  | □ No | □ Yes |  |  |  |
|  | Gloves and PPE worn by operators |  | □ No | □ Yes |  |  |  |
|  | Previous Item blended & Batch No : |  |  |  |  |  |  |
| Room Condition | Room Temperature (not more than 25degC) (ID. No.:________) Due Date: _______ |  | Relative Humidity (not more than 75%) |  |  |  |  |
| Equipment Check | Blending: | Multi-directional Mixer | □ EQP ID : BL -06 |  |  |  |  |
|  | Sieving (ID. No.) |  | □ #80 (ID. No.:_________) |  |  |  |  |
|  | Electronic Balance (60kgs) |  | □ EQP No: P030 □ EQP No: _____ |  |  |  |  |

Note: Production should not proceed unless all checks comply.

Line Clearance given by (Production Supervisor): ______________   Date: __________________

Page 3.4j

BLENDING RECORD

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
| 7.0 | Blending: | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 7.1 | Load the Metformin HCl dried granules of Lot-1, Lot-2 and Lot-3, into the multi directional mixer and set the blending time for 10 minutes in timer and start the operation by pressing “start” button on control panel. Refer SOP Operation of multi direction mixer- SWSG/PRD/0027. 将 Lot-1、Lot-2、Lot-3 的Suniton干燥颗粒装入多向混合器中，在定时器中设定混合时间为10分钟，按控制面板上的“开始”按钮开始操作。请参阅多向混合器的SOP操作 - SWSG/PRD/0027 Blending Time (mins) 混合时间（分钟）; Blender speed (Hz) 多向混合器转速; Start time 开始时间; End time 结束时间. Set time 设定时间; Actual time 实际时间. 10 mins 10分钟. |  |  |
|  | Note: Check the integrity of sieve/screen before and after sieving/milling activity and record the details in the below table. Sieve size/screen 筛网尺寸; Sieve/screen integrity before use 使用前筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. #80 mesh. |
| 7.2 | Sift the Magnesium Stearate manually through cleaned #80 mesh and collect in polyethylene bag lined container. 将Magnesium Stearate手动筛分通过清洁的#80筛网，并收集在衬有聚乙烯袋的容器中。 QC Reference No. QC 参考编号; Weight of Magnesium Stearate (kg) Magnesium Stearate重量 (kg); Sieve used 所用筛子. |  |  |
|  | Sieve size/screen 筛网尺寸; Sieve/screen integrity after use 使用后筛网的完整性. Satisfactory 满意; Not Satisfactory* 不满意; Remarks 评论; Checked by sign & date 检查者 签名和日期; Verified by qualified person sign & date 合格人员验证 签名和日期. #80 mesh. *Mention the details if the integrity of sieve/screen not satisfactory in the remarks column. *如果筛子的完整性不满意，请在备注栏中注明详细信息。 |

Page 3.4j

Page 3.4j

| Step | Manufacturing procedure |  |  |
| --- | --- | --- | --- |
| 8.0 | Lubrication: | Done by sign & date | Checked by sign & date |
| 8.1 | Collect approx. 15.0 kg of blended granules from multi directional mixer of step no. 7.1 into double polyethylene lined container. Add sifted magnesium stearate of step no. 7.2 material and premix manually. 从多向混合器收集来自步骤7.1大约15.0kg的混合颗粒并装入双层聚乙烯内衬容器中。添加步骤7.2的过筛magnesium stearate至的物料以及手动预混。 |  |  |
| 8.2 | Add the premix of step no. 8.1 into the multi directional mixer of step no. 7.1 and set the lubrication time for 5 minutes in timer and start the operation by pressing “start” button on control panel. Refer SOP Operation of multi direction mixer- SWSG/PRD/0027. 添加步骤号8.1的预混料进入步骤7.1的多向混合器和在定时器中设定润滑时间为5分钟，按控制面板上的“启动”按钮开始运行。请参阅多向混合器的SOP操作 - SWSG/PRD/0027 。 Blending Time (mins) 混合时间（分钟）; Blender speed (Hz) 多向混合器转速; Start time; End time. Set time 设定时间; Actual time 实际时间. 5 mins 5分钟. |  |  |
| 8.3 | Tare the blue drums with lid, double polyethylene bag, Cable tie and record the tare details on the label. 对带盖、双层聚乙烯袋、扎带的蓝色桶进行皮重称重，并在标签上记录皮重详细信息。 Unload the lubricated blend into tared double polyethylene bag lined containers. 将润滑后的混合物卸载到已称皮重的双层聚乙烯袋内衬容器中。 |  |  |
| 8.4 | Collect the lubricated blend pooled sample approx. 100 g from unloaded containers or drums into a zip lock bag and send to quality control (QC) for blend assay testing. 从容器或桶中收集约 100 克润滑混合物混合样品放入密封袋中，并送至质量控制 (QC) 进行混合物分析测试。 - Pooled sample 1 ( from drum 1 to 10) 混合样本 1 (从鼓 1 到 10) - Pooled sample 2 (from drum 11 to 19) 混合样本 2 (从鼓 11 到 19) |  |  |

Page 3.4j

Page 3.4k

Lubricated Granule Details

- Tare the blue drums with lid, double polyethylene bag, Cable tie and record the tare details on the label.

- Unload the lubricated blend into tared double polyethylene bag lined containers.

- Record the weight details in the table below and container label. And ensure gross weight should not exceed 25.0 kg.

| Drum No. 鼓号 | Tare weight (kg)-A (Weight of drum + polyethylene bag) | Gross weight (kg)-B (Weight of material + A) | Net weight (kg) (B-A) |
| --- | --- | --- | --- |
| Total weight (kg) |  |  |  |

Weighed by sign & date: ________________ Checked by sign & date: ________________

|  | To be completed by Production supervisor: Yield and Reconciliation. A; Theoretical Yield (Theoretical weight of blend); 377.78 kg. B; Actual weight of lubricated granules; __________kg. C; Percentage of Theoretical yield (B/A x 100); __________%. *Yield limit is 95.0 – 102 % Calculated by sign & date: ________________ Checked by sign & date: _____________________ |
| --- | --- |

Page 3.4k

Page 3.4l

In-Process QC Report

To be completed by QC Dept.

| Assay Sample | SI. No. | Approximate Qty (g) | Sign & Date | Time (am/pm) |
| --- | --- | --- | --- | --- |
|  | Pooled sample 1 (S1) (from drum no. _____to _____) |  |  |  |
|  | Pooled sample 2 (S2) (from drum no. _____to _____) |  |  |  |

| Test | Result |  | Specification |
| --- | --- | --- | --- |
| Description : |  |  | White granules |
| Bulk density |  |  | NA (Information purpose) |
| Tapped density |  |  | NA (Information purpose) |
| Assay: | S1 |  | 95.0 – 105.0% of Metformin HCI |
|  | S2 |  |  |

Unit weight used in assay

Result: Passed / Failed

Remarks:               _______________________________________________________________________________

_______________________________________________________________________________

_____________________	_____________________

Analysed By	Checked By

_____________________	_____________________

Date	Date

Page 4.1

Area Checklist

GMP Checklist:                                                                                 Date:                                           Time:

| TABLETING DEPARTMENT |  |  |  | Tick (√) where appropriate |  | Sign & Date By |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Done By | Production Supervisor |
| Work Area Checklist | Materials from previous batch are removed |  |  | - □ No □ Yes |  |  |  |
|  | Room Name: |  |  | □ Unclean □ Clean |  |  |  |
|  | Room is labelled |  |  | - □ No □ Yes Yes |  |  |  |
|  | Status of Utensils |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Vacuum Cleaner |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Tablet Press |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Deduster |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Exhaust |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Air-con |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Suction Hose |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Weighing Balance |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Hardness Tester, Equip id: ___________ |  |  | □ Unclean □ Clean |  |  |  |
|  | Status of Thickness Gauge, Equip id: __________ |  |  | □ Unclean □ Clean |  |  |  |
|  | All equipment are labelled |  |  | □ No □ Yes |  |  |  |
|  | Gloves and PPE worn by operators |  |  | □ No □ Yes |  |  |  |
|  | Previous Item compressed & Batch No: |  |  |  |  |  |  |
| Room Condition | Room Temperature (Not more than 25° C) (ID. No. ______) |  |  | Relative Humidity (Not more than 75%) |  |  |  |
|  | Tablet Press |  |  | □ TP-05 □ TP-07 |  |  |  |
|  | Electronic Balance (300 gms) |  |  | □ P020 □ P035 □ P037 |  |  |  |
|  | Electronic Balance (60 Kgs) |  |  | □ P040 □ P041 □ P042 |  |  |  |
|  | Punch Size & Description/Serial No |  |  |  |  |  |  |
| Equipment Check Placebo Granules |  | Batch No. |  | Quantity Used | Kg |  |  |
|  |  |  |  | Quantity Disposed | kg |  |  |

Note: Production should not proceed unless all checks comply.

To Production Personnel: Please be reminded to report immediately to the production management team whenever there

is any manufacturing deviation.

Line Clearance given by (Production Controller): ___________________              Date: ____________________

Page 4.2

TABLETTING REPORT

| 9.0 | Compression: |  |  |
| --- | --- | --- | --- |
| 9.1 | Set up the tablet compression machine with the following tooling. 使用以下工具设置安装压片机。 Size: 10 mm 尺寸: 10毫米 Punch shape: Round convex 冲头形状: 圆形扁平 Tooling type: D 模具类型: D Upper punch: 上冲头: Lower punch: 下冲头: No. of set:__________ 套数:_____________ | Done by sign & date 完成者 签名和日期 | Checked by sign & date 检查者 签名和日期 |
| 9.2 | Take the placebo granules (if applicable) and load them into tablet machine hopper and compress the placebo granules into tablets. Check the physical properties of the tablets. After placebo run clean the machine and discard the placebo tablets and granules if any. 取placebo颗粒（如适用）装入压片机料斗中，压制成片剂。检查片剂的物理性质。Placebo运行后，清洁机器并丢弃placebo片剂和颗粒（如果有）。 Note: Placebo granules compression only applicable after level 2 cleaning (Change product). 注意：Placebo颗粒压缩仅适用于2级清洁（更换产品）后。 Placebo granules quantity used: ______________kg. 使用的placebo颗粒数量：______________kg Placebo discarded: _______________kg. 丢弃的placebo：_______________kg Done by sign & date: ________________ Checked by sign & date: _________________ 完成者签名和日期：___________________检查者签名和日期：_______________________ |  |  |

Page 4.2

B. To be completed by the Production Pharmacist Only.

Total Weight of Granules Received                     	: _____________________________Kg

| Parameter | Target | Range |
| --- | --- | --- |
| Physical description | White Round, convex score |  |
| Theoretical Weight of Tablet (mg) | 359.79 mg |  |
| Individual Tablet Weight variation (mg) | 359.79 mg ± 5% | 341.80 – 377.78 mg |
| Weight of 10 Tablets (g) | 3.5979 g ± 2.5% | 3.5079 – 3.6878 g |
| Thickness (mm) | 4.3 – 5.0 mm |  |
| Hardness (kg) | 3.0 – 12 kg |  |
| Disintegration time (Mins) | NMT 30 minutes |  |
| Friability (%) | NMT 0.8% |  |

Final Compress Weight                                         	: ________________________mg/tab

Confirmed by Production Pharmacist               	: _________________________

Date                                                                         	: _________________________

Note:

- Initial IPQC checks should be done by QC Personnel.

- Initially after setting the machine, QC personnel should check all the IPQC checks of initial tablets and record it in

"RAW DATA FOR PHYSICAL TEST (IPQC)" FM/QP/027D.

- Once all the IPQC checks are satisfactory, production will start.

- Carry out the In-Process checks during tabletting as per the frequency mentioned in BMR by production operator.      - Check the sensitivity of Metal detector at start, after any stoppage or breakdown and at the end of each Batch.

Record the results in “Metal Detection Test Record Form” (FM/PT/EO/003A).

Attach the completed form with BMR for review.

Page 4.3a

In-Process Specification

To be complete by QC Dept:

Tablet Sample:

| Sl. No: | Approximate Qty (Nos) | Sign & Date | Time (am/pm) |
| --- | --- | --- | --- |

| DESCRIPTION | CURRENT | SPECIFICATIONS |
| --- | --- | --- |
| Physical |  | White Round, convex score |
| Diameter |  | 10 mm |
| Thickness |  | 4.3 – 5.0 mm |
| Hardness |  | 3 – 12 kg |
| Friability |  | NMT 0.8% |
| Disintegration Time |  | NMT 30 minutes |
| Dissolution Rate |  | NLT 75% (Q) in 45 minutes |
| Assay |  | 95.0 – 105.0% of Metformin HCI |

Result: Passed/ Failed

Remarks:               _______________________________________________________________________________

_______________________________________________________________________________

_____________________	_____________________

QC Department	Date

Page 4.3b

In-Process Check

Remark: All the initial IPQC checks are □ satisfactory / □ unsatisfactory      Done by IPQC (sign, date. time): ______________________________________

| Date | Time | Appearance of the tablets (Put OK or NOT OK) | Wt of 10 Tablets (g) | Hardness (kg) (3 - 12kg) |  |  |  |  |  |  |  |  |  | Thickness (mm) (4.3 – 5.0mm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

* Weight of 10 tablets every 15 mins and at each start of machine.                                                              Done by: ___________________    Checked by: __________________

* Appearance, Hardness & Thickness at every 30 mins and at each start of machine.                                                                (Sign & Date)                                       (Sign & Date)

Page 4.3b

In-Process Check

| Date | Time | Appearance of the tablets (Put OK or NOT OK) | Wt of 10 Tablets (g) | Hardness (kg) (3 - 12kg) |  |  |  |  |  |  |  |  |  | Thickness (mm) (4.3 – 5.0mm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

* Weight of 10 tablets every 15 mins and at each start of machine.                                                              Done by: ___________________    Checked by: __________________

* Appearance, Hardness & Thickness at every 30 mins and at each start of machine.                                                                (Sign & Date)                                       (Sign & Date)

Page 4.3b

In-Process Check

| Date | Time | Appearance of the tablets (Put OK or NOT OK) | Wt of 10 Tablets (g) | Hardness (kg) (3 - 12kg) |  |  |  |  |  |  |  |  |  | Thickness (mm) (4.3 – 5.0mm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

* Weight of 10 tablets every 15 mins and at each start of machine.                                                              Done by: ___________________    Checked by: __________________

* Appearance, Hardness & Thickness at every 30 mins and at each start of machine.                                                                (Sign & Date)                                       (Sign & Date)

Page 4.3b

In-Process Check

| Date | Time | Appearance of the tablets (Put OK or NOT OK) | Wt of 10 Tablets (g) | Hardness (kg) (3 - 12kg) |  |  |  |  |  |  |  |  |  | Thickness (mm) (4.3 – 5.0mm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

* Weight of 10 tablets every 15 mins and at each start of machine.                                                              Done by: ___________________    Checked by: __________________

* Appearance, Hardness & Thickness at every 30 mins and at each start of machine.                                                                (Sign & Date)                                       (Sign & Date)

Page 4.3b

In-Process Check

| Date | Time | Appearance of the tablets (Put OK or NOT OK) | Wt of 10 Tablets (g) | Hardness (kg) (3 - 12kg) |  |  |  |  |  |  |  |  |  | Thickness (mm) (4.3 – 5.0mm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

* Weight of 10 tablets every 15 mins and at each start of machine.                                                              Done by: ___________________    Checked by: __________________

* Appearance, Hardness & Thickness at every 30 mins and at each start of machine.                                                                (Sign & Date)                                       (Sign & Date)

Page 4.3b

In-Process Check

| Date | Time | Appearance of the tablets (Put OK or NOT OK) | Wt of 10 Tablets (g) | Hardness (kg) (3 - 12kg) |  |  |  |  |  |  |  |  |  | Thickness (mm) (4.3 – 5.0mm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

* Weight of 10 tablets every 15 mins and at each start of machine.                                                              Done by: ___________________    Checked by: __________________

* Appearance, Hardness & Thickness at every 30 mins and at each start of machine.                                                                (Sign & Date)                                       (Sign & Date)

| 9.3 | Load the actual product granules into the compression machine hopper using suction hose and start tableting as per parameters mentioned in BMR specifications. 使用吸料软管将实际产品颗粒装入压缩机料斗中，并按照 BMR 规范中提到的参数开始压片。 Parameters/Time 参数/时间; Spec 规格; Initial 初始. Machine speed (rpm) 机器转速; 30 - 45. Feeder speed (rpm) 给料机转速; -. Pre compression force (kN) 预压缩力; -. Main compression force (kN) 主压缩力; -. Punch penetration (mm) 冲头穿透力; -. Cam size (mm) 凸轮尺寸; -. Fill depth (mm) 填充深度; -. Note: Record the above parameters every 2 hours or when change in parameters in above table. 注意：每2小时或上表参数发生变化时记录一次以上参数。 |
| --- | --- |

Page 4.3b

Page 4.4

TABLETTING RECORDS

|  | Tableting record table shall be updated as below, 压片记录表应更新如下： Date 日期; Drum no. 鼓号; Net weight (kg) 净重; Average unit weight of a tablet (mg) 片剂的平均单位重量; Done by sign & date 完成者签名和日期. Total (B) 总数; kg; mg. Checked by Sign & date: _________________ 检查者签名和日期：________ __ To be completed by Production supervisor: Yield and Reconciliation. A; Theoretical Yield (Qty. of lubricated granules transferred for compression); __________kg. B; Actual yield (Qty. of acceptable tablets obtained); __________kg. C; Average unit weight of a tablet; ___________mg. D; Core tablets obtained in no’s (B/C x 1000000); ________________ no’s. E; Percentage of Theoretical yield (B/A x 100); __________%. F; Sample quantity for analysis (IPQC); __________kg. G; Loss from initial setup; __________kg. H; Rejects (Machine and tablet defects); __________kg. I; Others; __________kg. J; Total rejects [(F+G+H+I); __________kg. K; Reconciliation [(B+J)/A x 100]; __________%. *Yield limit is 95.0 – 102 % Calculated by sign & date: ___________________ Checked by sign & date: _______________________ |
| --- | --- |

Page 4.4

Page 5.3

Area Checklist

GMP Checklist: 	Date:                                  Time:

| COATING DEPARTMENT |  |  | Tick (√) where appropriate |  |  | Sign & Date By |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Coating Personnel | Production Supervisor |
| Work Area Check | Materials from previous batch are removed |  | □ No | □ Yes |  |  |  |
|  | Status of room, Room name: |  | □ Unclean | □ Clean |  |  |  |
|  | Room is labelled |  | □ No | □ Yes |  |  |  |
|  | Status of Preparation Tank |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Spray Nozzle |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Vacuum Cleaner |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Pump |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Coating Machine |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Exhaust |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Containers & Utensils |  | □ Unclean | □ Clean |  |  |  |
|  | Status of Air-con |  | □ Unclean | □ Clean |  |  |  |
|  | All equipment are labelled |  | □ No | □ Yes |  |  |  |
|  | Gloves and PPE worn by operators |  | □ No | □ Yes |  |  |  |
|  | Previous Item coated & Batch No: |  |  |  |  |  |  |
| Room Condition | Room Temperature (not more than 25 deg C) (ID No: ________) |  | Relative Humidity (Not more than 75%) |  |  |  |  |
| Equipment Check | FILM – COATIING MACHINE |  | □ EQP ID: FC - 02 |  |  |  |  |
|  | Electronic Balance (330 gms) |  | □ P027 |  |  |  |  |
|  | Electronic Balance (60 Kgs) |  | □ P044 |  |  |  |  |

Line Clearance given by (Production Supervisor): __________________         Date________________

Page 5.4a

Preparation Record

| Raw Materials | Theoretical Wt/Volume 50L | Actual Weight | Signed & Date |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Received By | Prepared By | Verified By |
| HPMC Methocel E15-LV | 1.75 kg | kg |  |  |  |
| Titanium Dioxide | 250 g | g |  |  |  |
| Calcium Carbonate Light BP | 150 g | g |  |  |  |
| Corn Starch | 50 g | g |  |  |  |
| Talcum Powder | 50 g | g |  |  |  |
| PEG 6000 | 500 g | g |  |  |  |
| Isopropyl Alcohol | (49L) 38.56 kg | kg |  |  |  |
| Purified Water (QC Rf. No:___________________) | 1 L | L |  |  |  |
| -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |
| -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |
| -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |
| -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |
| -- -- | -- -- | -- -- | -- -- | -- -- | -- -- |

Page 5.4b

COATING SOLUTION PREPARATION PROCEDURE

| 10.0 | Film coating (Coating solution preparation): |  |  |
| --- | --- | --- | --- |
| 10.1 | After equipment setting completes, inform production supervisor for coating material issuance. 设备安装完成后，通知生产主管发放涂料。 Production supervisor shall issue the coating materials from intermediate storage room as per planned manufacturing product ticket and transfer to the manufacturing area. 生产部主管根据计划的生产产品单从中间储藏室发放涂料并转运至生产区域。 Receive the dispensed coating materials from production supervisor and verify the materials, check the material label bag by bag for Material name, Quantity, Lot No., QC reference no., as per BWR and BMR page 5.4a. 从生产主管处接收分配的涂层材料并验证材料，按照 BWR 和 BMR 第 5.4a 页逐袋检查材料标签上的材料名称、数量、批号、QC 参考号， |  |  |
| 10.2 | Take cleaned 60L SS container and take the tare weight of container and pour 38.56kg (49L) dispensed Isopropyl alcohol (IPA), Dip the power mixer blade shaft into the IPA contain container start stirring to form vortex by adjusting variable speed button from 6 to 9 on power mixer and slowly add 1.75 kg of the HPMC Methocel E15-LV under stirring and continue the stirring till HPMC completely dissolve. 取已洗净的60L不锈钢容器并称出容器的皮重，倒入38.56kg（49L）分配的Isopropyl alcohol（IPA），将动力搅拌器叶片轴浸入IPA容器中，通过将在动力搅拌器上的变速按钮从6调节至9开始搅拌以形成涡流和在搅拌下缓慢加入1.75kg HPMC Methocel E15-LV，并继续搅拌直至HPMC完全溶解。 | Done by sign & date 完成者签名和日期 | Checked by sign & date 检查者签名和日期 |
| 10.3 | Add Titanium dioxide, Calcium carbonate light, Corn starch, and talcum powder into the step no. 10.2 under stirring and continue stirring for about 20 minutes or until a smooth homogeneous lump-free solution is formed. 在搅拌下将Titanium dioxide、Calcium carbonate light、Corn starch和talcum powder加入步骤10.2中，继续搅拌约20分钟或直至形成光滑均匀无块状的溶液。 |  |  |
| 10.4 | Take 1.0 L (1.0kg) purified water in separate SS vessel and slowly add PEG 6000 under stirring. Continue the stirring for about 10minutes or until homogeneous lump-free solution is formed. 取1.0 L (1.0kg) 纯净水于另一个不锈钢容器中，搅拌下缓慢加入PEG 6000。继续搅拌约10分钟或直至形成均匀无块状溶液。 |  |  |
| 10.5 | Add the PEG 6000 solution of step 10.4 into step 10.3 (HPMC solution) and continue stirring for about 10 minutes or until homogeneous solution is formed. 将步骤 10.4 的 PEG 6000 溶液加入步骤 10.3（HPMC 溶液）中，继续搅拌约 10 分钟或直至形成均匀溶液。 |  |  |
| 10.6 | Page 5.4b Filter the solution using #100 mesh into the spraying tank and weigh and record the coating suspension quantity. 使用#100目筛网将溶液过滤到喷涂罐中，称重并记录涂层悬浮液量。 Container number 容器号码; Tare Weight (kg) (A) 皮重 (kg) (A); Gross Weight (kg) (B) 毛重(kg) (B); Net Weight (kg) (B-A) 净重(kg) (B-A). Total weight (kg) 总重量 (kg). Done by sign & date: _______________________ Checked by sign & date:__________________ 完成者签名和日期：_______________________检查者签名和日期：___ |

Page 5.5

Film Coat Specifications

| 11.0 | Film coating |
| --- | --- |
| 11.1 | After coating solution preparation is complete, inform production supervisor for core tablets issuance. 包衣溶液制备完成后，通知生产部主管发放片芯。 Production supervisor shall issue the core tablets from intermediate storage room as per planned manufacturing product ticket and transfer to the coating area. 生产部主管应按照计划生产产品票从中间存储室发出片芯并转移到包衣区域。 Receive the core tablets from production supervisor and verify the batch no., ticket no., Quantity as per BMR. 从生产部主管处接收片芯，并根据 BMR 验证批次号、票号、数量。 Product name: _____________________Batch No.: _____________ Total no. of drums: _____________ 产品名称：____________________ _批号：________________ 总数鼓数：_____________ Total core tablets received quantity from tableting stage: ___________kg 华语 kg Divide the core tablets into approximately 6 equal lots and label the drums with lot no., quantity, batch no. and record in the table below. 将片芯分成大约 6 个等份，并在桶上贴上批号、数量、批号的标签，并记录在下表中。 Lot No. 批号; Quantity (kg) (Given by pharmacist) 数量-药剂师提供; Quantity taken by operator 操作员所取数量; Done by sign & date 完成者签名和日期; Checked by sign & date 检查者签名和日期. Lot 1. Lot 2. Lot 3. Lot 4. Lot 5. Lot 6. |

Page 5.5a

FILM COATING IN-PROCESS CHECKS

| 11.2 | Gun validation喷枪验证: After completion of coating solution preparation, set up the peristaltic pump and spray nozzles. 包衣溶液制备完成后，设置蠕动泵和喷嘴。 Take the 2 small beakers and take the tare weigh on electronic weighing balance. 取2个小烧杯，在电子天平上称取皮重。 Set the peristaltic pump speed as per BMR and hold the tared beakers below the spray nozzle and start the peristaltic pump. 根据 BMR设置蠕动泵速度，将称好重量的烧杯放在喷嘴下方并启动蠕动泵。 Collect the solution for about 60 seconds and stop the peristaltic pump. 收集溶液约60秒，停止蠕动泵。 Weigh the beaker with solution and record in the table below. Repeat the same steps for trial 2 and trial 3. 称量盛有溶液的烧杯，并记录在下表中。对第 2次试验和第3次试验重复相同步骤。 Gun Validation. Weight of total coating solution (kg). Parameters 参数; Trial 1 Peristaltic pump speed ________ 蠕动泵转速 Spray time (60 secs) ________ 喷涂时间(60 秒); Trial 2 Peristaltic pump speed ________ 蠕动泵转速 Spray time (60 secs) ________ 喷涂时间(60 秒); Trial 3 Peristaltic pump speed ________ 蠕动泵转速 Spray time (60 secs) ________ 喷涂时间(60 秒). Gun (1); Gun (2); Gun (1); Gun (2); Gun (1); Gun (2). Weight of empty measuring jar (A) (g) 空量瓶重量(A). Weight of Measuring Jar with Solution (B) (g) 盛有溶液 (B) 的量杯重量. Difference (A-B) spray rate (g/min/gun) 差值(AB)喷涂量. Note: Check the spray pattern by setting the spray rate, atomization air pressure as per BMR. Ensure spray should not overlap. 注意：根据 BMR 设置喷雾速率、雾化气压，检查喷雾模式。确保喷雾不重叠。 |
| --- | --- |

| 11.3 | Page 5.5a Load the core tablets of Lot-1 from step no. 11.1 into coating pan and setup the spray guns and coating parameters as per below mentioned. 将步骤 11.1的 Lot-1的核心片剂放入包衣锅并按以下提及的参数规格设置喷枪和包衣参数。 Parameter 参数; Limit限制. Target目标; Range范围. No. of spray gun 喷枪数量; 2. Spray nozzle size; 1.0 mm. Spray procedure 喷涂程序; Continuous. Pre warming 预热. Inlet temperature (°C) 入口温度; 60°C; 50°C - 70°C. Outlet temperature (°C) 出口温度; NA; 40°C ±5°C. Bed temperature (°C) 床层温度; 40°C; 35°C - 45°C. Pan speed (RPM) 平移速度; Jog mode; NA. Spraying/coating喷涂/涂装. Inlet temperature (°C) 入口温度; 60°C; 55°C - 65°C. Outlet temperature (°C) 出口温度; NA; 45°C ±5°C. Bed temperature (°C) 床层温度; 45°C; 45°C ±5°C. Pan speed (RPM) 平移速度; 2; 1 – 5 rpm. Spray rate (g/min/gun) 喷涂速率; 100; 90 – 110 g/min/gun. Peristaltic pump speed蠕动泵转速; 50; 40 - 60. Airflow (cfm); 2000 – 4500 cfm. Gun to Gun distance (cm) 枪与枪之间的距离; 16 – 18 cm. Gun to Bed distance (cm) 枪到床的距离; 15 – 25 cm. Atomization air pressure (kg/cm2) 雾化气压; 3.0; 2.0 – 4.0 kg/cm2. Drying干燥. Inlet temperature (°C) 入口温度; 60°C ±10°C. Pan speed (rpm) 平移速度; Jog mode. Drying time (mins) 干燥时间; 5 – 10 mins. Cooling冷却. Inlet temperature (°C) 入口温度; OFF. Pan speed (rpm) 平移速度; Jog mode. Cooling time (mins) 冷却时间; 15 minutes. Note: Adjust the coating parameters within above mentioned limits to achieve satisfactory film coating. 注意：在上述范围内调整包衣参数，以获得满意的包衣效果。 |
| --- | --- |
| 11.4 | Lot no. 批号; LOT-1; Page 5.5a Done by sign & date 完成者签名和日期; Verified by qualified person sign & date 合格人员验证签名和日期. Pan Load (kg): 盘负载 (kg):. Gun to Gun distance (cm) – 枪对枪距离(厘米) - Range: 16 – 18 cm 范围: 16 – 18 厘米. Gun to Bed distance (cm) – 喷枪到床的距离(厘米) – 范围：15 – 25厘米 Range: 15 – 25 cm. Gun angle (45°) 枪角度 (45°). |
| 11.5 | Pre warming: 预热： After adjustment are made, warm the tablets while jogging the pan until the tablet bed temperature reaches to 35°C - 45°C. Once bed temperature reaches to 35°C - 45°C, take some tablets to check for pre warmed tablets weight. 调整完成后，边慢摇锅边加热片剂，直至片床温度达到35°C - 45°C。一旦床温达到35°C - 45°C，取一些药片检查预热的药片重量。 Pre-warming of tablets- LOT-1 片剂预热 - LOT-1. Start time 开始时间; End time 结束时间; Total pre-warming time (min) 总预热时间(分钟); Inlet temp. (°C) 50°C - 70°C 入口温度(°C) 50°C–70°C; Bed temp. (°C) 35°C - 45°C 床层温度(°C) 35°C - 45°C; Outlet temp. (°C) 35°C - 45°C; Pan speed 平移速度. Pre warmed tab weight 20 tabs (g) 预热片重量20 片(g); sample 1 (20 tabs) g 样品1 (20 片)g; sample 2 (20 tabs) g 样品2 (20 片)g; sample 3 (20 tabs) g 样品3 (20 片)g; Average of 3 samples (A) g 3个样品的平均值(A) g; Weight of pre warmed tabs (B) [(Ax1000/20)] mg 预热片重量(B) [(Ax1000/20)]mg. |

Page 5.5a

| 11.6 | Start the coating pan and spray the film coating suspension on tablets, continue the spraying till the tablets are covered uniform film. Monitor and record the process parameters, weight gain details every 30 minutes. 启动包衣锅，将薄膜包衣悬浮液喷在药片上，继续喷洒，直至药片上覆盖均匀的薄膜。每30分钟监测并记录过程参数、重量增加的详细信息。 In-process parameters check- LOT-1 工艺参数检查- LOT-1. Time/ Parameters 时间/参数; Pan speed (1 -5 rpm) 平移速度; Inlet temp. (55 - 65°C) 入口温度; Outlet temp. (40 - 50°C) 出口温度; Bed temp. (40 - 50°C) 床层温度; Atomization air pressure (2.0 – 4.0 kg/cm2) 雾化气压; Peristaltic pump speed (40 – 60 rpm) 蠕动泵转速; Spray rate (90 – 110 g/min/gun) 喷涂速率; Airflow (2000 – 4500 cfm) 气流; Done by sign & date 完成者 签名和日期. |
| --- | --- |

Page 5.5a

| 11.7 | Page 5.5a Coating weight build-up record for Coating process-LOT-1 涂层工艺-LOT-1的涂层重量累积记录. Time 时间; Cumulative spray time (min) 累计喷雾时间(分钟); Weighing record; Average of 3 samples (C) g 3个样品的平均值(C)克; Weight of coated tab (D) [(Cx1000/20)] 涂层片重量(D) [(Cx1000/20)]; % weight gain per tab [(D-B/B) x 100] 每片增重百分比 [(D-B/B) x 100]; Done by sign & date 完成者 签名和日期. sample 1 (20 tabs) g 样品1 (20片)克; sample 2 (20 tabs) g 样品2 (20片)克; sample 3 (20 tabs) g 样品3 (20片)克. |
| --- | --- |
| 11.8 | After completion of coating, dry the tablets at inlet temperature to 50 - 70°C, while jogging the pan for 5 – 10 minutes and record the time. 包衣完成后，将片剂在入口温度干燥至 50 - 70°C，同时慢跑锅 5 - 10 分钟并记录时间。 Total drying time: ___________ 总干燥时间：___________ |
| 11.9 | Cooling冷却: After completion of drying, Allow the tablets to cool down up to 15 minutes with inlet temperature heater OFF. 干燥完成后，在入口温度加热器关闭的情况下，让片剂冷却长达 15 分钟。 While jogging the pan and record the time. 同时让平底锅慢跑并记录时间。 Total cooling time: ___________ 总冷却时间:______ Note: Repeat the same procedure from step 11.4 to 11.9 for remaining lots 注意：对剩余批次重复步骤 11.4 至 11.9 的相同程序 |

| 11.4 | Lot no. 批号; LOT-2; Page 5.5a Done by sign & date 完成者签名和日期; Verified by qualified person sign & date 合格人员验证签名和日期. Pan Load (kg): 盘负载 (kg):. Gun to Gun distance (cm) – 枪对枪距离(厘米) - Range: 16 – 18 cm 范围: 16 – 18 厘米. Gun to Bed distance (cm) – 喷枪到床的距离(厘米) – 范围：15 – 25厘米 Range: 15 – 25 cm. Gun angle (45°) 枪角度 (45°). |
| --- | --- |
| 11.5 | Pre warming: 预热： After adjustment are made, warm the tablets while jogging the pan until the tablet bed temperature reaches to 35°C - 45°C. Once bed temperature reaches to 35°C - 45°C, take some tablets to check for pre warmed tablets weight. 调整完成后，边慢摇锅边加热片剂，直至片床温度达到35°C - 45°C。一旦床温达到35°C - 45°C，取一些药片检查预热的药片重量。 Pre-warming of tablets- LOT-2 片剂预热 - LOT-2. Start time 开始时间; End time 结束时间; Total pre-warming time (min) 总预热时间(分钟); Inlet temp. (°C) 50°C - 70°C 入口温度(°C) 50°C–70°C; Bed temp. (°C) 35°C - 45°C 床层温度(°C) 35°C - 45°C; Outlet temp. (°C) 35°C - 45°C; Pan speed 平移速度. Pre warmed tab weight 20 tabs (g) 预热片重量20 片(g); sample 1 (20 tabs) g 样品1 (20 片)g; sample 2 (20 tabs) g 样品2 (20 片)g; sample 3 (20 tabs) g 样品3 (20 片)g; Average of 3 samples (A) g 3个样品的平均值(A) g; Weight of pre warmed tabs (B) [(Ax1000/20)] mg 预热片重量(B) [(Ax1000/20)]mg. |

Page 5.5a

| 11.6 | Start the coating pan and spray the film coating suspension on tablets, continue the spraying till the tablets are covered uniform film. Monitor and record the process parameters, weight gain details every 30 minutes. 启动包衣锅，将薄膜包衣悬浮液喷在药片上，继续喷洒，直至药片上覆盖均匀的薄膜。每30分钟监测并记录过程参数、重量增加的详细信息。 In-process parameters check- LOT-2 工艺参数检查- LOT-2. Time/ Parameters 时间/参数; Pan speed (1 -5 rpm) 平移速度; Inlet temp. (55 - 65°C) 入口温度; Outlet temp. (40 - 50°C) 出口温度; Bed temp. (40 - 50°C) 床层温度; Atomization air pressure (2.0 – 4.0 kg/cm2) 雾化气压; Peristaltic pump speed (40 – 60 rpm) 蠕动泵转速; Spray rate (90 – 110 g/min/gun) 喷涂速率; Airflow (2000 – 4500 cfm) 气流; Done by sign & date 完成者 签名和日期. |
| --- | --- |

Page 5.5a

| 11.7 | Page 5.5a Coating weight build-up record for Coating process-LOT-2 涂层工艺-LOT-2的涂层重量累积记录. Time 时间; Cumulative spray time (min) 累计喷雾时间(分钟); Weighing record; Average of 3 samples (C) g 3个样品的平均值(C)克; Weight of coated tab (D) [(Cx1000/20)] 涂层片重量(D) [(Cx1000/20)]; % weight gain per tab [(D-B/B) x 100] 每片增重百分比 [(D-B/B) x 100]; Done by sign & date 完成者 签名和日期. sample 1 (20 tabs) g 样品1 (20片)克; sample 2 (20 tabs) g 样品2 (20片)克; sample 3 (20 tabs) g 样品3 (20片)克. |
| --- | --- |
| 11.8 | After completion of coating, dry the tablets at inlet temperature to 50 - 70°C, while jogging the pan for 5 – 10 minutes and record the time. 包衣完成后，将片剂在入口温度干燥至 50 - 70°C，同时慢跑锅 5 - 10 分钟并记录时间。 Total drying time: ___________ 总干燥时间：___________ |
| 11.9 | Cooling冷却: After completion of drying, Allow the tablets to cool down up to 15 minutes with inlet temperature heater OFF. 干燥完成后，在入口温度加热器关闭的情况下，让片剂冷却长达 15 分钟。 While jogging the pan and record the time. 同时让平底锅慢跑并记录时间。 Total cooling time: ___________ 总冷却时间:______ Note: Repeat the same procedure from step 11.4 to 11.9 for remaining lots 注意：对剩余批次重复步骤 11.4 至 11.9 的相同程序 |

| 11.4 | Lot no. 批号; LOT-3; Page 5.5a Done by sign & date 完成者签名和日期; Verified by qualified person sign & date 合格人员验证签名和日期. Pan Load (kg): 盘负载 (kg):. Gun to Gun distance (cm) – 枪对枪距离(厘米) - Range: 16 – 18 cm 范围: 16 – 18 厘米. Gun to Bed distance (cm) – 喷枪到床的距离(厘米) – 范围：15 – 25厘米 Range: 15 – 25 cm. Gun angle (45°) 枪角度 (45°). |
| --- | --- |
| 11.5 | Pre warming: 预热： After adjustment are made, warm the tablets while jogging the pan until the tablet bed temperature reaches to 35°C - 45°C. Once bed temperature reaches to 35°C - 45°C, take some tablets to check for pre warmed tablets weight. 调整完成后，边慢摇锅边加热片剂，直至片床温度达到35°C - 45°C。一旦床温达到35°C - 45°C，取一些药片检查预热的药片重量。 Pre-warming of tablets- LOT-3 片剂预热 - LOT-3. Start time 开始时间; End time 结束时间; Total pre-warming time (min) 总预热时间(分钟); Inlet temp. (°C) 50°C - 70°C 入口温度(°C) 50°C–70°C; Bed temp. (°C) 35°C - 45°C 床层温度(°C) 35°C - 45°C; Outlet temp. (°C) 35°C - 45°C; Pan speed 平移速度. Pre warmed tab weight 20 tabs (g) 预热片重量20 片(g); sample 1 (20 tabs) g 样品1 (20 片)g; sample 2 (20 tabs) g 样品2 (20 片)g; sample 3 (20 tabs) g 样品3 (20 片)g; Average of 3 samples (A) g 3个样品的平均值(A) g; Weight of pre warmed tabs (B) [(Ax1000/20)] mg 预热片重量(B) [(Ax1000/20)]mg. |

Page 5.5a

| 11.6 | Start the coating pan and spray the film coating suspension on tablets, continue the spraying till the tablets are covered uniform film. Monitor and record the process parameters, weight gain details every 30 minutes. 启动包衣锅，将薄膜包衣悬浮液喷在药片上，继续喷洒，直至药片上覆盖均匀的薄膜。每30分钟监测并记录过程参数、重量增加的详细信息。 In-process parameters check- LOT-3 工艺参数检查- LOT-3. Time/ Parameters 时间/参数; Pan speed (1 -5 rpm) 平移速度; Inlet temp. (55 - 65°C) 入口温度; Outlet temp. (40 - 50°C) 出口温度; Bed temp. (40 - 50°C) 床层温度; Atomization air pressure (2.0 – 4.0 kg/cm2) 雾化气压; Peristaltic pump speed (40 – 60 rpm) 蠕动泵转速; Spray rate (90 – 110 g/min/gun) 喷涂速率; Airflow (2000 – 4500 cfm) 气流; Done by sign & date 完成者 签名和日期. |
| --- | --- |

Page 5.5a

| 11.7 | Page 5.5a Coating weight build-up record for Coating process-LOT-3 涂层工艺-LOT-3的涂层重量累积记录. Time 时间; Cumulative spray time (min) 累计喷雾时间(分钟); Weighing record; Average of 3 samples (C) g 3个样品的平均值(C)克; Weight of coated tab (D) [(Cx1000/20)] 涂层片重量(D) [(Cx1000/20)]; % weight gain per tab [(D-B/B) x 100] 每片增重百分比 [(D-B/B) x 100]; Done by sign & date 完成者 签名和日期. sample 1 (20 tabs) g 样品1 (20片)克; sample 2 (20 tabs) g 样品2 (20片)克; sample 3 (20 tabs) g 样品3 (20片)克. |
| --- | --- |
| 11.8 | After completion of coating, dry the tablets at inlet temperature to 50 - 70°C, while jogging the pan for 5 – 10 minutes and record the time. 包衣完成后，将片剂在入口温度干燥至 50 - 70°C，同时慢跑锅 5 - 10 分钟并记录时间。 Total drying time: ___________ 总干燥时间：___________ |  |
| 11.9 | Cooling冷却: After completion of drying, Allow the tablets to cool down up to 15 minutes with inlet temperature heater OFF. 干燥完成后，在入口温度加热器关闭的情况下，让片剂冷却长达 15 分钟。 While jogging the pan and record the time. 同时让平底锅慢跑并记录时间。 Total cooling time: ___________ 总冷却时间:______ Note: Repeat the same procedure from step 11.4 to 11.9 for remaining lots 注意：对剩余批次重复步骤 11.4 至 11.9 的相同程序 |  |
| 11.4 |  | Lot no. 批号; LOT-4; Page 5.5a Done by sign & date 完成者签名和日期; Verified by qualified person sign & date 合格人员验证签名和日期. Pan Load (kg): 盘负载 (kg):. Gun to Gun distance (cm) – 枪对枪距离(厘米) - Range: 16 – 18 cm 范围: 16 – 18 厘米. Gun to Bed distance (cm) – 喷枪到床的距离(厘米) – 范围：15 – 25厘米 Range: 15 – 25 cm. Gun angle (45°) 枪角度 (45°). |
| 11.5 |  | Pre warming: 预热： After adjustment are made, warm the tablets while jogging the pan until the tablet bed temperature reaches to 35°C - 45°C. Once bed temperature reaches to 35°C - 45°C, take some tablets to check for pre warmed tablets weight. 调整完成后，边慢摇锅边加热片剂，直至片床温度达到35°C - 45°C。一旦床温达到35°C - 45°C，取一些药片检查预热的药片重量。 Pre-warming of tablets- LOT-4 片剂预热 - LOT-4. Start time 开始时间; End time 结束时间; Total pre-warming time (min) 总预热时间(分钟); Inlet temp. (°C) 50°C - 70°C 入口温度(°C) 50°C–70°C; Bed temp. (°C) 35°C - 45°C 床层温度(°C) 35°C - 45°C; Outlet temp. (°C) 35°C - 45°C; Pan speed 平移速度. Pre warmed tab weight 20 tabs (g) 预热片重量20 片(g); sample 1 (20 tabs) g 样品1 (20 片)g; sample 2 (20 tabs) g 样品2 (20 片)g; sample 3 (20 tabs) g 样品3 (20 片)g; Average of 3 samples (A) g 3个样品的平均值(A) g; Weight of pre warmed tabs (B) [(Ax1000/20)] mg 预热片重量(B) [(Ax1000/20)]mg. |

Page 5.5a

| 11.6 | Start the coating pan and spray the film coating suspension on tablets, continue the spraying till the tablets are covered uniform film. Monitor and record the process parameters, weight gain details every 30 minutes. 启动包衣锅，将薄膜包衣悬浮液喷在药片上，继续喷洒，直至药片上覆盖均匀的薄膜。每30分钟监测并记录过程参数、重量增加的详细信息。 In-process parameters check- LOT-4 工艺参数检查- LOT-4. Time/ Parameters 时间/参数; Pan speed (1 -5 rpm) 平移速度; Inlet temp. (55 - 65°C) 入口温度; Outlet temp. (40 - 50°C) 出口温度; Bed temp. (40 - 50°C) 床层温度; Atomization air pressure (2.0 – 4.0 kg/cm2) 雾化气压; Peristaltic pump speed (40 – 60 rpm) 蠕动泵转速; Spray rate (90 – 110 g/min/gun) 喷涂速率; Airflow (2000 – 4500 cfm) 气流; Done by sign & date 完成者 签名和日期. |
| --- | --- |

Page 5.5a

| 11.7 | Page 5.5a Coating weight build-up record for Coating process-LOT-4 涂层工艺-LOT-4的涂层重量累积记录. Time 时间; Cumulative spray time (min) 累计喷雾时间(分钟); Weighing record; Average of 3 samples (C) g 3个样品的平均值(C)克; Weight of coated tab (D) [(Cx1000/20)] 涂层片重量(D) [(Cx1000/20)]; % weight gain per tab [(D-B/B) x 100] 每片增重百分比 [(D-B/B) x 100]; Done by sign & date 完成者 签名和日期. sample 1 (20 tabs) g 样品1 (20片)克; sample 2 (20 tabs) g 样品2 (20片)克; sample 3 (20 tabs) g 样品3 (20片)克. |
| --- | --- |
| 11.8 | After completion of coating, dry the tablets at inlet temperature to 50 - 70°C, while jogging the pan for 5 – 10 minutes and record the time. 包衣完成后，将片剂在入口温度干燥至 50 - 70°C，同时慢跑锅 5 - 10 分钟并记录时间。 Total drying time: ___________ 总干燥时间：___________ |  |
| 11.9 | Cooling冷却: After completion of drying, Allow the tablets to cool down up to 15 minutes with inlet temperature heater OFF. 干燥完成后，在入口温度加热器关闭的情况下，让片剂冷却长达 15 分钟。 While jogging the pan and record the time. 同时让平底锅慢跑并记录时间。 Total cooling time: ___________ 总冷却时间:______ Note: Repeat the same procedure from step 11.4 to 11.9 for remaining lots 注意：对剩余批次重复步骤 11.4 至 11.9 的相同程序 |  |
| 11.4 |  | Lot no. 批号; LOT-5; Page 5.5a Done by sign & date 完成者签名和日期; Verified by qualified person sign & date 合格人员验证签名和日期. Pan Load (kg): 盘负载 (kg):. Gun to Gun distance (cm) – 枪对枪距离(厘米) - Range: 16 – 18 cm 范围: 16 – 18 厘米. Gun to Bed distance (cm) – 喷枪到床的距离(厘米) – 范围：15 – 25厘米 Range: 15 – 25 cm. Gun angle (45°) 枪角度 (45°). |
| 11.5 |  | Pre warming: 预热： After adjustment are made, warm the tablets while jogging the pan until the tablet bed temperature reaches to 35°C - 45°C. Once bed temperature reaches to 35°C - 45°C, take some tablets to check for pre warmed tablets weight. 调整完成后，边慢摇锅边加热片剂，直至片床温度达到35°C - 45°C。一旦床温达到35°C - 45°C，取一些药片检查预热的药片重量。 Pre-warming of tablets- LOT-5 片剂预热 - LOT-5. Start time 开始时间; End time 结束时间; Total pre-warming time (min) 总预热时间(分钟); Inlet temp. (°C) 50°C - 70°C 入口温度(°C) 50°C–70°C; Bed temp. (°C) 35°C - 45°C 床层温度(°C) 35°C - 45°C; Outlet temp. (°C) 35°C - 45°C; Pan speed 平移速度. Pre warmed tab weight 20 tabs (g) 预热片重量20 片(g); sample 1 (20 tabs) g 样品1 (20 片)g; sample 2 (20 tabs) g 样品2 (20 片)g; sample 3 (20 tabs) g 样品3 (20 片)g; Average of 3 samples (A) g 3个样品的平均值(A) g; Weight of pre warmed tabs (B) [(Ax1000/20)] mg 预热片重量(B) [(Ax1000/20)]mg. |

Page 5.5a

| 11.6 | Start the coating pan and spray the film coating suspension on tablets, continue the spraying till the tablets are covered uniform film. Monitor and record the process parameters, weight gain details every 30 minutes. 启动包衣锅，将薄膜包衣悬浮液喷在药片上，继续喷洒，直至药片上覆盖均匀的薄膜。每30分钟监测并记录过程参数、重量增加的详细信息。 In-process parameters check- LOT-5 工艺参数检查- LOT-5. Time/ Parameters 时间/参数; Pan speed (1 -5 rpm) 平移速度; Inlet temp. (55 - 65°C) 入口温度; Outlet temp. (40 - 50°C) 出口温度; Bed temp. (40 - 50°C) 床层温度; Atomization air pressure (2.0 – 4.0 kg/cm2) 雾化气压; Peristaltic pump speed (40 – 60 rpm) 蠕动泵转速; Spray rate (90 – 110 g/min/gun) 喷涂速率; Airflow (2000 – 4500 cfm) 气流; Done by sign & date 完成者 签名和日期. |
| --- | --- |

Page 5.5a

| 11.7 | Page 5.5a Coating weight build-up record for Coating process-LOT-5 涂层工艺-LOT-5的涂层重量累积记录. Time 时间; Cumulative spray time (min) 累计喷雾时间(分钟); Weighing record; Average of 3 samples (C) g 3个样品的平均值(C)克; Weight of coated tab (D) [(Cx1000/20)] 涂层片重量(D) [(Cx1000/20)]; % weight gain per tab [(D-B/B) x 100] 每片增重百分比 [(D-B/B) x 100]; Done by sign & date 完成者 签名和日期. sample 1 (20 tabs) g 样品1 (20片)克; sample 2 (20 tabs) g 样品2 (20片)克; sample 3 (20 tabs) g 样品3 (20片)克. |
| --- | --- |
| 11.8 | After completion of coating, dry the tablets at inlet temperature to 50 - 70°C, while jogging the pan for 5 – 10 minutes and record the time. 包衣完成后，将片剂在入口温度干燥至 50 - 70°C，同时慢跑锅 5 - 10 分钟并记录时间。 Total drying time: ___________ 总干燥时间：___________ |  |
| 11.9 | Cooling冷却: After completion of drying, Allow the tablets to cool down up to 15 minutes with inlet temperature heater OFF. 干燥完成后，在入口温度加热器关闭的情况下，让片剂冷却长达 15 分钟。 While jogging the pan and record the time. 同时让平底锅慢跑并记录时间。 Total cooling time: ___________ 总冷却时间:______ Note: Repeat the same procedure from step 11.4 to 11.9 for remaining lots 注意：对剩余批次重复步骤 11.4 至 11.9 的相同程序 |  |
| 11.4 |  | Lot no. 批号; LOT-6; Page 5.5a Done by sign & date 完成者签名和日期; Verified by qualified person sign & date 合格人员验证签名和日期. Pan Load (kg): 盘负载 (kg):. Gun to Gun distance (cm) – 枪对枪距离(厘米) - Range: 16 – 18 cm 范围: 16 – 18 厘米. Gun to Bed distance (cm) – 喷枪到床的距离(厘米) – 范围：15 – 25厘米 Range: 15 – 25 cm. Gun angle (45°) 枪角度 (45°). |
| 11.5 |  | Pre warming: 预热： After adjustment are made, warm the tablets while jogging the pan until the tablet bed temperature reaches to 35°C - 45°C. Once bed temperature reaches to 35°C - 45°C, take some tablets to check for pre warmed tablets weight. 调整完成后，边慢摇锅边加热片剂，直至片床温度达到35°C - 45°C。一旦床温达到35°C - 45°C，取一些药片检查预热的药片重量。 Pre-warming of tablets- LOT-6 片剂预热 - LOT-6. Start time 开始时间; End time 结束时间; Total pre-warming time (min) 总预热时间(分钟); Inlet temp. (°C) 50°C - 70°C 入口温度(°C) 50°C–70°C; Bed temp. (°C) 35°C - 45°C 床层温度(°C) 35°C - 45°C; Outlet temp. (°C) 35°C - 45°C; Pan speed 平移速度. Pre warmed tab weight 20 tabs (g) 预热片重量20 片(g); sample 1 (20 tabs) g 样品1 (20 片)g; sample 2 (20 tabs) g 样品2 (20 片)g; sample 3 (20 tabs) g 样品3 (20 片)g; Average of 3 samples (A) g 3个样品的平均值(A) g; Weight of pre warmed tabs (B) [(Ax1000/20)] mg 预热片重量(B) [(Ax1000/20)]mg. |

Page 5.5a

| 11.6 | Start the coating pan and spray the film coating suspension on tablets, continue the spraying till the tablets are covered uniform film. Monitor and record the process parameters, weight gain details every 30 minutes. 启动包衣锅，将薄膜包衣悬浮液喷在药片上，继续喷洒，直至药片上覆盖均匀的薄膜。每30分钟监测并记录过程参数、重量增加的详细信息。 In-process parameters check- LOT-6 工艺参数检查- LOT-6. Time/ Parameters 时间/参数; Pan speed (1 -5 rpm) 平移速度; Inlet temp. (55 - 65°C) 入口温度; Outlet temp. (40 - 50°C) 出口温度; Bed temp. (40 - 50°C) 床层温度; Atomization air pressure (2.0 – 4.0 kg/cm2) 雾化气压; Peristaltic pump speed (40 – 60 rpm) 蠕动泵转速; Spray rate (90 – 110 g/min/gun) 喷涂速率; Airflow (2000 – 4500 cfm) 气流; Done by sign & date 完成者 签名和日期. |
| --- | --- |

Page 5.5a

| 11.7 | Page 5.5a Coating weight build-up record for Coating process-LOT-6 涂层工艺-LOT-6的涂层重量累积记录. Time 时间; Cumulative spray time (min) 累计喷雾时间(分钟); Weighing record; Average of 3 samples (C) g 3个样品的平均值(C)克; Weight of coated tab (D) [(Cx1000/20)] 涂层片重量(D) [(Cx1000/20)]; % weight gain per tab [(D-B/B) x 100] 每片增重百分比 [(D-B/B) x 100]; Done by sign & date 完成者 签名和日期. sample 1 (20 tabs) g 样品1 (20片)克; sample 2 (20 tabs) g 样品2 (20片)克; sample 3 (20 tabs) g 样品3 (20片)克. |
| --- | --- |
| 11.8 | After completion of coating, dry the tablets at inlet temperature to 50 - 70°C, while jogging the pan for 5 – 10 minutes and record the time. 包衣完成后，将片剂在入口温度干燥至 50 - 70°C，同时慢跑锅 5 - 10 分钟并记录时间。 Total drying time: ___________ 总干燥时间：___________ |
| 11.9 | Cooling冷却: After completion of drying, Allow the tablets to cool down up to 15 minutes with inlet temperature heater OFF. 干燥完成后，在入口温度加热器关闭的情况下，让片剂冷却长达 15 分钟。 While jogging the pan and record the time. 同时让平底锅慢跑并记录时间。 Total cooling time: ___________ 总冷却时间:______ Note: Repeat the same procedure from step 11.4 to 11.9 for remaining lots 注意：对剩余批次重复步骤 11.4 至 11.9 的相同程序 |

Page 5.6a

FILM COATING RECORD

|  | Unload the tablets into tared blue drums lined with double polyethylene bags, label the containers, and record the weight of coated tablets. 将药片卸载到已称重的内衬双层聚乙烯袋的蓝色桶中，贴上标签，并记录包衣药片的重量。 Date 日期; Lot no. 批号; Drum no. 鼓号; Net weight(kg) 净重; Average unit weight of a tablet (mg) 剂片的平均重量; Done by sign & date 完成者签名和日期. Total weight (kg) B4总重量. Checked by Sign & date: _________________ 检查者签名和日期：_______________ |
| --- | --- |
|  | Send the remaining quantity of coating suspension for destruction. 将剩余的涂层悬浮液送去销毁。 Quantity of coating solution sent for destruction: _____________kg 送去销毁的涂层溶液数量：_____________kg |

Page 5.6b

FILM COATING RECORD

Note: Intimate IPQC to collect sample for QC analysis

Intimation given by: _________________                    Intimation received by: _________________

(Production)          (Sign & date)                                                         (IPQC)       (Sign & date)

To be completed by Production Supervisor:

| Qty of compressed tablets transferred for coating(B3): | =_________(kg)+3.3kg (Coating Solution) = _________(kg) |
| --- | --- |
| Qty. of film coated tablets obtained (B4) | = __________(kg) |
| Qty. of samples drawn as samples for IPQC (C2) | = __________(kg) |
| Total Qty. of rejects (D2) | = __________(kg) |
| Reconciliation | =B4+C2+D2 B3×100% = _______________×100% (C2): = _______________% |
| Qty of core tablets obtained | =Qty of acceptable tablets obtained B4 Average unit weight of a tablet×1,000,000 =_________________×1,000,000 =_________________No.s (B4) . |
| Yield (Limits: 95.0-102.0%) | =B4 in No.s Batch Size×100% =XXXXXXX1,050,000'S×100% =__________ % |

Done By/Date: ______________________                             Checked By/Date: _____________________

Page 5.7

FINISHED PRODUCT SPECIFICATION

To be complete by QC Dept:

Tablet sample:

| Sl. No. | Approximate Qty. (No.s) | Sign & Date | Time (am/pm) |
| --- | --- | --- | --- |

| DESCRIPTION | CURRENT | SPECIFICATION |
| --- | --- | --- |
| Physical |  | White Round, convex score |
| Diameter |  | 10mm |
| Thickness |  | 4.5 – 5.2 mm |
| Disintegration Time |  | NMT 30 minutes |
| Dissolution Rate |  | NLT 75% (Q) in 45 minutes |

Result: Passed/ Failed

Remarks:

___________________________________________            	________________________

___________________________________________                         	QC Department

___________________________________________             	________________________		Date